*****************************************

*                                       *

*       TandyLink / 3Com Mail Bridge    *

*                                       *

*           Setup Instructions          *

*                                       *

*****************************************

The following instructions outline the steps involved in setting up

and installing the hardware and software needed to operate a Mail

Bridge between the TandyLink and 3Com networks. If you have already

performed any of the steps, simply check to be sure there is no

conflict between what you have done and the corresponding steps in

these instructions.

1.  Install both network adapter boards in the Mail Bridge machine.

==================================================================

See the installation manuals for the TandyLink adapter and the 3Com

adapter for details of jumper positions. There are potential

conflicts with other option boards (such as the hard disk board), so

you might also need the manuals for your other option boards.

HARDWARE INTERRUPTS. Set the interrupts so that neither adapter board

interferes with the other adapter board, the hard disk, or any other

devices or option boards in the machine. Possible settings include:

Hard Disk=2, TandyLink=3, 3Com=5 (on a Tandy 1000 TX)

TandyLink=2, 3Com=3, Hard Disk=5 (on a Tandy 3000)

If you have an EtherLink II adapter board, you can set the interrupts

through software and need not set them at this stage.

DMA CHANNELS. On the TandyLink board, you must set the DMA channels to

DMA=1 because the hard drive will require DMA=3.

If you use an EtherLink 3C501 board, you can avoid DMA conflict by

setting the DMA type to Programmed I/O loop. Pass a fourth parameter

"2" to the ETH.SYS driver in CONFIG.SYS, and it will not use DMA.

The sample CONFIG.SYS file below shows ETH.SYS loaded with

Interrupt 3, and DMA Type 2 (none).

If you use an EtherLink II 3C503 board, pass the parameter "/m:2" to

the ETH503.SYS driver, again using Programmed I/O and not DMA.

If you use an EtherLink Plus 3C505 board, use DMA channel 5, 6, or 7.

Computers that do not have DMA channels 5 through 7 cannot use the

3C505 board. The Tandy 3000 HL, for example, should have the 3C501

or 3C503 installed if used as a mail bridge machine.

I/O BASE ADDRESS. Set this address only if you have special software

or hardware that conflicts with I/O base address 398 (the default I/O

base address for the TandyLink board) or the default I/O base address

of your EtherLink board.

When you are satisfied that the jumpers on the adapter boards are

suitably set, install both boards in empty slots in the Mail Bridge

machine.

2.  Make the network connections.

=================================

Connect each board to the appropriate network. The connections will be

standard TandyLink and 3Com EtherLink connections.

3.  Install a 3Com User called "Bridge."

========================================

Using the standard 3Com installation, set up the Mail Bridge machine

as a workstation under 3Com. You must install 3Com 3+ Release 1.2 or

higher and install the NetBIOS option on the workstation. Create a

3Com user called "Bridge." You can specify a 3+ Domain as in

"bridge : domain". Bridge is the recommended user name. Be sure the

Bridge is given a valid default 3+Mail Server.

To allow for EtherLink board settings, you might need to modify the

CONFIG.SYS file. The following sample could be used for an EtherLink

3C501 board set for Hardware Interrupt 3 and DMA=none.

shell=c:\command.com /P /E:384

device=c:\sys\pro.sys 32 2 2

device=c:\sys\eth.sys 3 x x 2

device=c:\sys\buf.sys

device=c:\sys\idp.sys

device=c:\sys\spp.sys

device=c:\sys\lgl.sys

lastdrive=g

buffers=6

files=20

If an EtherLink II 3C503 board were being used, the third line of

the CONFIG.SYS file could be:

device=c:\sys\eth503.sys /i:3 /m:2

If an EtherLink Plus 3C505 board were being used, and DMA=7, the

third line of the CONFIG.SYS file would be:

device=c:\sys\eth505.sys 3 x 7

In the above example, C:\SYS is a hard disk directory that contains

the necessary .sys files. The SHELL= statement gives the default

command processor (COMMAND.COM) an expanded environment space, to

ensure that required environment variables can be entered.

You can place the 3Com program files in any directory you wish.

However, the following instructions and sample files assume that

you chose the directory C:\3COM for 3Com executable programs such

as RUNMINDS.COM and LOGIN.EXE.

After installing the 3Com software, reboot the Mail Bridge machine

to initialize the device drivers. Then, run 3Com. At this point,

AUTOEXEC.BAT should be similar to this:

runminds mindspro mindsbuf mindseth mindsidp mindsspp mindslgl

netbios /c /d2

minses12

msredir /z:4096

setname $$3com$$

Log in as Bridge, and run 3+Mail to ensure that Bridge can send and

receive 3+Mail. Log out of 3Com, and return to the system prompt.

Before proceeding to the next step you will need to delete the

AUTOEXEC.BAT file that you created and reboot the machine.

* * * * Tandy 1000SL and 1000TL Users * * * * *

If you are using a Tandy 1000SL or 1000TL for your Mail Bridge system,

please note that you must boot your system from disk rather than booting

up from ROM.

4.  Install DeskMate and DeskMate Workgroup Companion.

======================================================

The directory names below are suggestions. If you use different names,

be sure to modify all occurrences of each name you change.

Make a directory such as C:\DESK to contain DeskMate software.

Copy all the files from each of the DeskMate diskettes into C:\DESK.

Then, copy all the files from the DeskMate Workgroup Companion

diskette into the same directory.

Make the directories C:\CFG and C:\MSG. C:\CFG will be used to store

the DeskMate configuration files, including DMLINK.CFG, which contains

the TandyLink configuration. C:\MSG will be the current working

directory when the Mail Bridge is operating. It will temporarily store

Mail messages in transit from one network to the other and will be

empty when messages have been successfully transmitted.

Set up the environment for an initial run of DeskMate by typing:

set dmconfig=c:\cfg [ENTER]

set path=c:\desk [ENTER]

cd c:\desk [ENTER]

Run DeskMate by typing:

desk [ENTER]

At DeskMate's Desktop screen you need to run the Workgroup setup program.

Select Run from the File Menu (F2), and run the program WGSETUP.PDM.

Set up a Workgroup client called "Bridge" to run on the TandyLink

network. You should accept the default settings, unless the jumper

settings you used on the Tandylink board (Step 1) were different from

the defaults of INT=2, DMA=1, I/O Base Address=398. Exit DeskMate,

return to the system prompt, and reboot.

Set up the environment for a second run of DeskMate by typing:

set dmconfig=c:\cfg [ENTER]

set path=c:\desk [ENTER]

cd c:\desk [ENTER]

Initialize the TandyLink network by typing:

wrkgroup [ENTER]

Run DeskMate by typing:

desk [ENTER]

At DeskMate's Desktop screen you need to run TandyLink Mail.

Select Run from the File Menu, and run Mail.pdm. Create mail, and send

a test message To: Bridge. Since you should have been logged in as

Bridge, you can now use Get Mail to retrieve the mail just sent.

5.  Prepare for Mail Bridge operation.

=======================================

The following batch file is designed for use as AUTOEXEC.BAT

on the Mail Bridge machine:

echo off

echo

echo DeskMate Mail Bridge Autoexec.bat

echo

echo TandyLink/3Com         08-18-1988

echo

rem Change to directory containing 3Com executables

rem

C:

cd \3Com

runminds mindspro mindsbuf mindseth mindsidp mindsspp mindslgl

netbios /c /d2

minses12

msredir /z:4096

setname $$3com$$

rem

rem Change to directory containing DeskMate executables

rem

C:

cd \Desk

set prompt=Bridge $p $g

set 3+TKM=14, 32

apimail

netswdrv

netsw 3com /in C:\3Com\login.exe Bridge

netsw 3com /out C:\3Com\logout.exe

netsw 3com /login

if errorlevel 1 goto 3Cfail

netsw 3com /logout

goto 3ComOK

:3Cfail

echo ERROR! your 3Com Login as Bridge failed

goto end

:3ComOK

netsw tandylink /in

netsw tandylink /out

set DMCONFIG=c:\cfg

set PATH=c:\Dos;c:\Bin;c:\Desk;c:\3Com;c:\Batch

wrkgroup

netsw tandylink /set

if errorlevel 1 goto TLfail

goto TLinkOK

:TLfail

echo ERROR! your TandyLink Login as Bridge failed

goto end

:TLinkOK

C:

cd \msg

echo Run DMBRIDGE.PDM as soon as possible !

pause

desk

rem

echo Please restart DeskMate Mail Bridge as soon as possible !

rem

:end

The file can be used without modification. However, you might need to change the following items if you used names that are different from those from those used in these instructions.

C:              The drive expected to contain all the required

directories.

\SYS            The directory containing the files used by CONFIG.SYS.

\3COM           The directory containing 3Com's executable programs.

\DESK           The directory containing the DeskMate and Workgroup

executable files.

\CFG            The directory containing DeskMate's configuration

files and certain DeskMate data files.

\MSG            The directory for temporary message storage.

Bridge          The 3Com login name for the Mail Bridge.

DMCONFIG=       An environment variable pointing to the directory

(C:\CFG) containing DeskMate's configuration files and

certain DeskMate data files.

PATH=           An environment variable that lists the directories in

which the 3Com and DeskMate executable files can be

found.

desk            When using a Tandy 1000 SL or 1000 TL this may need to

be x:desk where x: is the ROM drive of the system.

AUTOEXEC.BAT and CONFIG.SYS on the Mail Bridge should conform

to the example files here, with any changes you need to make.

6.  Test the Mail Bridge operation.

===================================

Rebooting should now allow test operation of the Mail Bridge.

The CONFIG.SYS and AUTOEXEC.BAT files should load 3Com's drivers,

load the software that performs the switching between networks, load the

TandyLink network, and lead into DeskMate. The DeskMate program

that performs the transfer of mail between the networks is

DMBRIDGE.PDM. Instructions for the operation of the program are

included in Appendix A of the Workgroup Companion documentation.

Messages regarding the state of the Mail Bridge system will

appear almost as soon as DMBRIDGE starts running. Error messages might

indicate malfunction of a portion of the system. You might need to

double-check the steps already taken.

7.  Send test mail from one network to the other.

=================================================

Once the Mail bridge is operating, you can leave the machine

operating more or less unattended. To send mail to the other network,

you address it To: the user Bridge on your network and include the

names of the intended recipients in a Relay: field. Instructions are

given in the Mail section of the DeskMate Workgroup Companion

documentation for sending mail between networks.

Since the users on one network do not see the users on the other

network, it would be useful to send a user list to the other network.

On a TandyLink machine, use DeskMate Mail to send test mail to each

3Com Mail user. Include a TandyLink mail user list and brief

instructions about sending mail between the networks. You can get a

list of TandyLink mail users by taking a directory of

\DMLINK\POSTOFF on the TandyLink server named Wrkgroup. Valid users

names will be the same as the individual directory names.

On a 3Com machine, use 3+Mail to send test mail to each TandyLink Mail

user. Include a 3Com mail user list and (if required) brief

instructions about sending mail between 3Com and TandyLink.

The DeskMate program NAMELIST produces output to a file NAMELIST.DOC,

which contains valid 3Com user names.

Within 15 minutes (the default time between Bridge operations),

the bridge should have retrieved the test mail and relayed it to the

users on the other network.