Tandy DAC Sound Package

Version 1.8

September 30, 1996

Send comments, bug reports to:

Jeffrey L. Hayes

130 Forest Ave., Lot 1

Orono, Me. 04473

(207) 866-7903

Internet:  tvdog@agate.net

Contents

--------

1.  Introduction

2.  Playwav

3.  Recwav

4.  Ppwav

5.  Raw2wav

6.  Snd2sam

7.  Setdac

8.  Piano

Introduction

------------

As noted in the file Tandac.doc (included in this package),

there is a general lack of software support for the built-in

digital-to-analog converter included by Tandy with several

models of the 1000 and 2500.  Generally speaking, only Tandy

and some game programmers have written programs for the chip.

The few programs that support the Tandy DAC include DeskMate

Music and Sound, which were bundled with the machine, and a

Windows 3.1 sound driver for the 2500-series, available from

Radio Shack (this driver will also work on the 1000RSX).

This package is intended to serve two goals:  first, to provide

a few useful programs; and second, to provide information on

the chip to encourage programmers to support it.  There is no

point in trying to redo something that has already been done

better than I could do it, however.  This package, therefore,

needs several free, shareware, and, yes, in one case commercial

(Sound.pdm) programs available separately to be most useful.

Here are some of them:

Sox, the universal sound sample translator, is an extremely

useful program, so useful that it is a "must have" if you

intend to do anything with sound.  Sox can convert most any

sound sample format you are likely to encounter (except, of

course, Tandy .snd :-).  It can also perform several effects on

the sound file, including adjusting the sampling rate, mixing

channels, high-, low- and bandpass filtering, adjusting the

volume, and echo and vibrato effects.  This program is

freeware.

MultiPlayer is a music module player that supports a wide

variety of sound cards; unfortunately, the Tandy sound chip is

not one of them.  MultiPlayer can play many module formats,

including MOD, 6CM, 8CM, WOW, STM, S3M, 669, OKT, MED, and P16.

This program requires a 286; module players that don't require

a 386 are scarce.  MultiPlayer's usefulness to Tandy owners

lies in the fact that it can play modules to a DAC on a

parallel port and to disk.  It can also play to the PC speaker,

but this is not recommended since the sound quality is

extremely poor.  With Setdac (described below), you can play

modules in real time on the Tandy DAC, but the quality will be

mediocre (at least on a 286) because of the low sampling rate

you can use.  To get better quality, select "Raw sample file"

as the output device, convert the module into an 8-bit unsigned

sample file at up to 22kHz, use Raw2wav (below) to convert the

.raw to a .wav, and use Playwav (also below) to play the .wav.

MultiPlayer is freeware also.  NOTE:  DO NOT USE MULTIPLAYER

V2.11.  THAT VERSION HAS A BUG IN THE COVOX DRIVER.

ModEdit is a music module editor for 4-channel Amiga module

files.  4-channel .mod's are the least common denominator among

standard music module types; any module player in existence,

for whatever platform, should be able to play them (although

the module format originates with the Amiga, it is now an

industry standard).  You can use Setdac to listen to your

modules while you edit them.  With Snd2sam, you can use

Sound.pdm to create and edit .mod samples, including looped

samples.  ModEdit is shareware; registration is $15.

Conv2snd, by Ken Udut, is a utility to convert RIFF WAVE and

other sound file formats to .snd so that they can be edited

with Tandy's Sound.pdm sound editor.  It comes with another

program, Snd2wav, which converts .snd to .wav, so that sounds

edited with Sound.pdm can be shared with people on non-Tandy

machines.  This program is freeware and comes with Turbo Pascal

source.  (I did the modifications to v2.00 of Conv2snd.)

Playsnd, by John Ball, plays Tandy .snd, Creative Labs .voc,

.wav, .au, Amiga .iff, and headerless unsigned PCM files (such

as Mac .snd) on the Tandy DAC or PC speaker.  The author also

has a program, Sbplay, to play the same types of sound files on

the Soundblaster, PC speaker, or a wide range of other

soundcards (using the Digpak driver set).  Both programs are

free.

David Stewart's Sampler package records and plays raw

headerless samples at near 11kHz and features voice activation.

The registered version purportedly provides keyboard activation

(nice because the voice activation seems to have a will of its

own) and allows the sampling rate to be varied.  This program

is shareware.

Sean Haynes' Tndsound program records and plays sound files in

its own proprietary format.  The unregistered version of this

package does not appear to be crippled.  There is a menu-driven

interface.  This program offers sound filtering and control

over the playback volume.  Overall, I would say that Tndsound

is better than Sampler.  One drawback to this program is that

it doesn't know how to compute the sampling rate in Hz from the

DAC divider value, so it's hard to select a standard sampling

rate (you have to specify the rate as a meaningless number).

The default rate is around 9kHz.  This program is shareware.

John W. Ratcliffe's IBM Digitized Sound Package ("Digpak")

contains a TSR, Tansltl.com, and a program, Play.exe, to play

sounds in a batch file.  There are a lot of features, including

MIDI on supported soundcards (the Tandy DAC/3-voice is,

unfortunately, not one).  It is mainly a programmer's package,

but the demo is interesting, and he seems to have a few things

available that users can use.

Tandy's own Sound.pdm deserves special mention.  Sound allows

you to record, play, and edit sound files, using a graphical

interface (the sound waveform is displayed).  The more I see of

Sound.pdm, the more I like it.  Sound.pdm runs under Tandy's

proprietary DeskMate interface, which is one of its limita-

tions; on some systems, DeskMate is in ROM and is lost when one

disables the ROM to upgrade the DOS version (DOS 3.3 is also in

ROM).  The other limitation is that sound files edited with

Sound.pdm must fit in conventional RAM - but there is still a

great deal of use for the program for short files, such as .mod

samples (see Snd2sam).  This program was bundled with several

Tandy models; perhaps they will sell you a copy at Radio Shack

if you tell them you have a 1000TL (or 1000RSX if you have that

or a 2500) and you lost your disk (the standard fee was $7.95

at last word).  If you don't have DeskMate, you can order the

generic IBM-compatible version of DeskMate 3.05, also at Radio

Shack (I don't know the current price).

The Music Transcription System is a music teaching and pitch

training system that can play and print musical scores and

display the pitch that you are singing at.

It should be mentioned that if you plan on using the sound

capabilities of the Tandy, you should get a pair of amplified

speakers (~$25-30) at your local Radio Shack and plug them into

the earphone jack on your computer.  The built-in speaker does

not produce very good sound.

WARNING:  CONNECTING AN AMPLIFIED INPUT (DIRECT LINE FROM

RADIO, ETC.) TO THE MICROPHONE JACK ON YOUR COMPUTER CAN DAMAGE

THE SOUND CIRCUITRY.  If you want to connect a stereo, tape

deck, or other device directly to the computer's sound input

via a dubbing cable, you *must* set a jumper inside the

machine.  Check your owner's manual before connecting any

amplified input - otherwise you will fry the sound chip and

possibly more.

This version includes eight programs, six of which are (hope-

fully) useful, one (Piano) which is a toy, and one (Testdiv)

which is mainly for programmers.  Most of the programs are

described in the following sections; see Tandac.doc for infor-

mation about Testdiv.

Playwav

-------

Playwav plays .wav files on the Tandy sound chip.  For sound

files in other formats, use Sox (see above) to convert to .wav.

The RIFF WAVE format, introduced with Windows version 3.0, is

among the most common sound formats out there.  Playwav

requires about 128k of free RAM (it works best with 256k on

1000SL's, TL's, and RL's with less than 768k RAM installed).

Arbitrarily large sound files can be played (the file is read

from disk while it's being played).

The WAVE format specification describes many types of sound

files and is a very flexible format.  However, the simplest

type of .wav format is by far the most common, and that is what

Playwav supports.  Specifically, with this program you can play

.wav files in Microsoft PCM format consisting of a single data

block with up to 16 bits per channel in mono or stereo, at

sampling rates up to 65535Hz.  That encompasses over 90% of all

.wav files in existence.  (One format that is increasingly

common is Microsoft ADPCM - Playwav does not recognize that

format.)

To use the program, just type its name at the DOS command line

followed by the name of the .wav file to be played, e.g.:

PLAYWAV FORCE_WI.WAV

On my system, this will play a sound file from Star Wars that

says "Remember, the Force will be with you, always."  If the

.wav is not in the current directory, include the drive and/or

path.  If the extension is not specified, it defaults to .wav;

to specify a file without an extension, end its name with a

period.

Playwav will then play the sound file.  If you want to use it

in a batch file, it returns ERRORLEVELs between 0 and 8.  Each

ERRORLEVEL is associated with a message.  Here are the

messages:

Playing - press a key to abort.

As it says, you can abort playback at any time.  This message

actually appears before the first bit of sound is loaded, so if

you're playing from a floppy drive you might be able to stop it

before it starts :-).

Done.

If nothing went wrong during playback, Playwav displays this

when the sound is done playing, or when you hit a key to abort.

In this case, the program returns ERRORLEVEL 0.

Insufficient memory.

This message indicates that Playwav was unable to allocate the

buffers it needed to play sound.  You might get this message if

you try to run Playwav when you're shelled out of something.

The program returns ERRORLEVEL 1 in this case.

Tandy DAC not found.

You get this message if you attempt to run this program on a

computer that does not have a Tandy sound chip.  See Tandac.doc

for a list of systems that have the DAC chip.  The 3-voice tone

and noise generator included with older 1000-series models is

not sufficient, and you will get this message if you try

Playwav on one of those.  (There are a couple .wav players for

the 3-voice on my ftp site.)  Playwav returns ERRORLEVEL 2.

Must specify input .wav file.

You get this message if you do not specify a .wav file to be

played on the command line.  Playwav returns ERRORLEVEL 3.

Open failed on .wav file.

You get this message if the file you specified does not exist.

There is also a remote possibility that open may fail for

another reason, such as a sharing conflict on a network.

Playwav returns ERRORLEVEL 4.

Invalid .wav file or unsupported type.

This message indicates that the .wav file you specified does

not have a .wav header indicating that it is of a type that

Playwav can play.  Either the file you specified is not a .wav

file, or the file is corrupt, or it is one of the rare types of

.wav files that the program does not support.  This message is

also displayed for Microsoft PCM .wav files that have more than

16 bits per sample or more than 2 channels, or that are sampled

at more than 65535 Hz.  Playwav returns ERRORLEVEL 5.

File or disk error reading .wav file.

You get this message if DOS reports an error reading from the

file after it has been opened.  You may have a bad spot on your

disk, or the disk structure may be corrupted; run CHKDSK or

SCANDISK.  Playwav returns ERRORLEVEL 6.

.wav file is truncated.

The end of the .wav file is missing.  This happens a lot to

files that get transferred over the Internet.  Playwav will

play as much of the file as there is, then return ERRORLEVEL 7.

Output underflow - unable to maintain sampling rate.

Copy .wav to hard disk or RAM disk and try again,

convert to 8-bit mono, or convert to reduce sampling

rate.

This message indicates that your computer is unable to generate

samples as fast as they need to be played.  Although Playwav is

written in hand-optimized assembler, your hardware may limit

the types of .wav's you can play.  Playwav was written and

tested on a 1000TL.  On this machine, I have played a 16-bit

stereo 44kHz .wav from the hard drive, but the best that I can

do from a floppy is 8-bit mono 11kHz.  The SL is a slower

machine and may not be able to do as well.  If you get this

message, you can use Ppwav to convert the .wav so that you can

play it (see below).  Playwav returns ERRORLEVEL 8.

Recwav

------

Recwav records .wav files on the Tandy sound chip.  The program

requires about 128k of free RAM (it works best with 256k on

1000SL's, TL's and RL's with less than 768k RAM installed).

Arbitrarily large sound files can be recorded, subject to disk

space limitations (the file is written to disk while it

records).  The output file is a RIFF WAVE file in 8-bit mono

Microsoft PCM format.  **SEE THE WARNING ABOVE REGARDING

AMPLIFIED INPUT.**

To use the program, type its name, followed by optional

switches, followed by the name of the output file, for example:

RECWAV MINABIRD

RECWAV /C ASOUND.

RECWAV /30000 C:\SOUNDS\SLOWSND

RECWAV /N NOCHECK.WAV

The following options are recognized:

/N tells Recwav to skip adjusting the sampling rate.  The

Tandy DAC can only record at a discrete set of rates, and by

default Recwav will look for an available rate close to the

specified one (or to the default rate of 11000Hz if you don't

specify a rate).  The .wav header will be adjusted to match the

rate chosen.

If /N is specified, on the other hand, Recwav will attempt

to select an available rate close to the one specified, but it

will not adjust the rate or change the .wav header.  The

adjustment is pretty fast, taking only a second or two (if

starting with the proper estimate for the model involved).

Recwav will still start up a little faster with /N, but sounds

will play back closer to the true pitch if /N is not specified.

/N will mainly be useful in cases where the system timer

has been speeded up from the default rate of 18.2 ticks per

second by some TSR.  To do its adjustment, Recwav must take

over the timer tick interrupt; when done, Recwav changes the

rate back to the default rate since there is no way to know if

the timer was speeded up before.  Likely culprits here are GW-

Basic and multi-tasking environments such as DesqView; in those

cases /N is mandatory.

/C tells Recwav to calibrate the microphone before

recording.  You are prompted to plug in the microphone but

remain silent, and the program records a short snippet of sound

to determine the correct baseline sample.  All samples

subsequently recorded are adjusted for the baseline.  Adjusting

for the baseline will improve the quality of the recorded

sound.

/nnnn is an optional sampling rate in Hz; for example,

/11025 or /44100.  A rate between 875 and 65535 may be

specified.  The default rate is 11000.

Higher sampling rates generally imply better sound quality

(up to a point), at the cost of greater disk space - the rate

is equal to the number of bytes per second of sound that will

be written to your disk.  The default rate is adequate for

speech.  If recording music, you may want to specify a higher

rate.  Standard rates are 5513Hz, 11025Hz, 22050Hz, and

44100Hz, but you can select any rate in the allowed range.

When selecting a sampling rate, note that your disk must be

fast enough to accept data at the specified rate.  This is most

likely to be a problem when recording to a floppy.  If your

disk can't keep up, Recwav will abort recording, display a

message (see below), and delete the output file.  On a 1000TL

with a reasonably fast hard drive, I can use the full range (up

to about 63kHz), but when recording to a floppy, the available

rate drops to less than 9000Hz.

As noted above, Recwav will normally adjust the rate you

specify to match one of the sound chip's available rates.

The output filename is required and can include drive and path

if you want.  If you do not specify an extension, it defaults

to .wav; to specify a file without an extension, end its name

with a period.

When you run Recwav, it displays the following:

I:\>recwav /8000 a:test

Adjusting sampling rate....

Recording RIFF WAVE file at 8082Hz.

Press a key to begin recording.

If /N is specified, you would see:

I:\>recwav /n /8000 a:test

Sampling rate not adjusted.

Recording RIFF WAVE file at 8000Hz.

Press a key to begin recording.

(Note that in this case it probably won't really record at

8000Hz, but something close; it just would put 8000Hz in the

.wav header.)  When ready to begin, press any key.  Recwav will

then say:

Press a key to stop recording.

Hit a key when you want to stop.  If all went well, the program

will display:

Done.

... and Recwav will return you to the DOS prompt (it returns

ERRORLEVEL 0 in this case).

If /C is specified, you get this:

I:\>recwav /c /8000 a:test

Adjusting sampling rate....

Recording RIFF WAVE file at 8082Hz.

Calibrating for the baseline sample.  If using a microphone, connect

it and remain silent.  If connecting a keyboard, radio, or tape deck

via a dubbing cable, connect the cable and turn the device on with the

volume off.  Press a key when ready.

Do what it says and hit a key.  The calibration is almost

instantaneous.  You will then be prompted to hit a key to start

recording, as above.

If something goes wrong, one of the following error messages

will be displayed:

Insufficient memory.

Recwav was unable to allocate a buffer for recording.  You are

most likely to get this if you run Recwav when shelled out of

something.  The program returns ERRORLEVEL 1 in this case.

Tandy DAC not found.

This message will be displayed if Recwav fails to detect the

needed sound chip in your system.  See Tandac.doc for a list of

the systems that have it.  The 3-voice tone and noise generator

included with older 1000 models is not sufficient.  The program

returns ERRORLEVEL 2.

Syntax:

RECWAV [/N] [/C] [/nnnn] <filename>

where:

/N     do not adjust recording rate

/C     calibrate microphone

/nnnn  recording rate in Hz (875-64000)

See TSPAK.DOC for details.

This message is displayed if you invoke Recwav without

parameters, or if the parameters you specify are invalid.  At

least one parameter (the output filename) is required.  The

program returns ERRORLEVEL 3.

Unable to create .wav file.

The output file you specified could not be created.  A likely

cause is trying to record to a write-protected diskette.

ERRORLEVEL 4 is returned.

File or disk error writing .wav file.

DOS reported an error while writing to the output file.  Recwav

intercepts critical errors, so you get this message if you have

a bad spot on your disk, as well as if the file structure on

the disk is corrupted.  A partial output file may have been

written, but Recwav can't safely delete it in this case.  Run

CHKDSK or SCANDISK to check the disk structure.  ERRORLEVEL 5

is returned.

Disk full - recording halted.

This message is displayed if a full disk is detected while

recording.  The output file is normally kept in this case.  You

get a valid .wav file as large as will fit on the disk.  If

Recwav was unable to write at least 2048 bytes, though, the

output file will be deleted.  ERRORLEVEL 6 is returned.

Input overflow - unable to maintain sampling rate.  Record to a hard disk

or RAM disk, or specify a lower rate.  Recording deleted.

This message indicates that your disk drive is unable to accept

data as fast as the sound chip is supplying it.  The output

file is deleted.  As noted above, you are most likely to get

this message when recording to a floppy.  Recwav returns

ERRORLEVEL 7.

Note that overflow will not occur if you record less than

64k of sound, so you can record short snippets of sound at a

higher rate than your disk would otherwise allow.

Ppwav

-----

Ppwav (preprocess .wav) performs 3 types of conversions of a

.wav file to reduce its size.  All of these conversions can be

done with Sox (see above), but Ppwav is much faster and easier

to use.  Ppwav needs about 53k of free RAM for its buffers.

To use Ppwav, type its name followed by the input file name and

the output file name, e.g.:

PPWAV PING1.WAV PG.WAV

On my system, this will start up Ppwav on PING1.WAV, a 16-bit

stereo 44kHz .wav and create a new sound file, PG.WAV.  (In

case you're wondering, PING1.WAV is a sonar ping taken from the

movie "Hunt for Red October.")

Ppwav will then display some statistics on the input file, for

example:

Input file:  PING1.WAV

2 channels

44100 samples per second (Hz)

16 bits per channel

samples are signed

It will then ask you what conversions you want to do:

Mix to mono? (Y/n)

Convert to 8-bit? (Y/n)

Halve sampling rate? (y/N)

The first two conversions are the default, as indicated by the

uppercase "Y".  They will be done unless you enter "n" or "N"

at the prompt.  The last conversion will not be done unless you

enter "y" or "Y".  The conversion may take a couple minutes.

All of these conversions will reduce the size of the file and

make it easier for Playwav to play it if you got the "Output

underflow" message.  (That message is the reason Ppwav exists.)

With the Tandy DAC, mixing to mono and converting to 8-bit do

not affect the sound quality since Playwav and other players

have to do that anyway to play the sound; 8-bit mono sound is

all the Tandy chip supports.  Halving the sampling rate will

adversely affect the sound quality, however, and you should

only do this as a last resort.  I find that 22kHz is about the

minimum for acceptable music, and 11kHz for speech.  Halving

the sampling rate is done by discarding every other sample.

If you haven't noticed, standard sampling rates are 5.5kHz,

11kHz, 22kHz and 44kHz.  44kHz 16-bit stereo is what audio CD's

are recorded at.  Both Playwav and Ppwav will handle other

sampling rates, however.  Sun .au files are recorded at 8kHz,

and Playwav will play them after Sox converts them to .wav.

Ppwav can also handle some rare .wav types that Playwav does

not:  sampling rates greater than 65535Hz, more than 2

channels, and more than 16 bits per channel.  At least I think

it will, it's supposed to.  I've never seen a .wav like that.

It will not handle all possible .wav's, however (in particular,

it will not handle compressed .wav's).

Ppwav doesn't display anything on the screen unless something

goes wrong.  Here are the error messages:

Usage: PPWAV <input file> <output file>

- see docs for details.

You get this message if you don't specify an input file and an

output file on the command line.  There is no default for the

output file; you have to specify it.

Error opening input file.

You get this message if the file you specified as the input

file does not exist.  As with Playwav, there is a remote

possibility of some other cause, but probably you mistyped.

Unable to create output file.

DOS reported that the output file could not be created.

Probably the root directory is full on the disk you specified

(if you specified the root directory), or the disk is full.

Error reading input .wav file.

DOS reported an error reading the input file.  This probably

means that the disk structure is corrupt, and you should run

Chkdsk to find out.  Unlike Playwav, Ppwav does not replace the

default critical error handler, so if you have a bad spot on

the disk, you will get the infamous "Abort, Retry, Ignore,

Fail?" message instead of this one.

Error writing output .wav file.

The disk structure is probably corrupt.  Ppwav ignores a full

disk; you just get as much of the output file as will fit.

(Playwav will play the fragment, but other players may not.)

Input .wav file invalid or unsupported type.

The input file specified is not a .wav or is corrupt, or it is

one of the rare .wav types that Ppwav does not recognize.

Raw2wav

-------

Raw2wav converts 8-bit unsigned raw sample files to .wav by

attaching a valid .wav header so that Playwav can play them.

This program is intended for use with MultiPlayer (see above).

MultiPlayer can convert music modules when "Raw sample file" is

specified as the output device.  Since the .raw files produced

by MultiPlayer are typically several megabytes in size, this

program overwrites the original .raw file in order to save disk

space.  Sox can do this conversion, but it is substantially

slower and does not overwrite, so you need double the disk

space of the original file.  (Raw2wav can also convert Mac .snd

or other types of headerless sound files.)

Since the original file is overwritten, it will be corrupted if

anything goes wrong.  Copy the file first if you're worried

about that.

To use Raw2wav, type its name, optionally followed by the

sampling rate, followed by the name of the .raw file to be

converted, e.g.:

RAW2WAV FUGUE.RAW

or:

RAW2WAV /11000 ENTRTAIN.RAW

The sampling rate defaults to 22000Hz if not specified; this is

the highest sampling rate MultiPlayer supports, and its use is

recommended.  Since the .raw file has no header, it is

impossible for a player to know how fast the file is to be

played if you don't specify it.  In the examples given,

FUGUE.RAW is the raw sample file produced by MultiPlayer when

playing FUGUE.MOD (Bach's organ fugue in D minor) to the Raw

sample file device at 22kHz, and ENTRTAIN.RAW is the file

produced by MultiPlayer when playing ENTRTAIN.MOD (Scott

Joplin's "The Entertainer") to the Raw sample file at 11kHz.

Raw2wav will ask you to confirm that you want to overwrite the

input file:

The input file will be overwritten.  Continue? (Y/n)

As the uppercase "Y" indicates, the default is "yes" if you

just hit return; Raw2wav will go on to convert the file.  The

input file will be renamed with a ".wav" extension.  If you

enter "n" or "N", the following message will be displayed:

Conversion not done.

and the program will terminate without changing the file.

Normally that will be the extent of the user interaction.  If

something goes wrong, one of the following error messages will

be displayed:

Must specify input .raw file.

You get this message if you don't specify an input file.

Sampling rate must be an integer in the range 875-65535.

You get this message if you specify a sampling rate that is not

a number, or if the number you specify is not in the indicated

range.  The Tandy sound chip cannot handle sampling rates lower

than 875Hz.  Rates higher than 65535Hz are technically

possible, but the programs in this package (except Ppwav) do

not support those rates because I don't want to deal with 32-

bit arithmetic on a 16-bit machine.  (None of these programs

contain 80386 instructions, or 80286 instructions for that

matter.)  If you actually have a .raw file sampled that high

(where did you get it?) you can use Sox to convert it.

Error opening input file for reading.

The input file specified does not exist, or some other error

occurred (unlikely).

Seek failed on input file.

DOS reported that it was unable to seek on the input file.  The

input file must be a file, not a device, since Raw2wav needs to

determine its size for the .wav header.  A corrupt disk

structure may also cause this error.

Error opening input file for writing.

DOS reported that the file could not be opened for writing.

This probably means that the file is read-only; use Attrib to

remove the read-only attribute.

Error reading from file.

This probably indicates a corrupt disk structure.  As with

Ppwav, the default critical error handler is not replaced, so

if you have a bad spot on the disk you will get "Abort, Retry,

Ignore, Fail?" instead of this.

Error writing to file.

DOS reported an error writing to the file.  I don't know how

this can happen.  A write-protected disk would cause a critical

error instead.

Disk full.

Since only 44 bytes are added to the file during the

conversion, this error is very unlikely (unless, perhaps, you

completely filled your disk when converting the .mod with

Multiplayer).  Nevertheless, there it is.

Insufficient memory.

Since Raw2wav is structured as a .com file rather than an .exe,

it must check for itself whether it has enough room to work in.

Since Ppwav is an .exe file, DOS checks whether there is enough

memory available before loading it, so in its case you would

get an error message from DOS, not from the program itself.

DOS only guarantees that Raw2wav gets memory equal to its file

size; it needs 32k more and checks whether it has it.

Snd2sam

-------

Snd2sam converts the .snd files produced by Sound.pdm (see

above) to .mod samples (.sam files) for use with ModEdit (see

above) or other Amiga music module editors.  Using Snd2sam, you

can use Tandy's excellent Sound editor program to create and

edit .mod samples, and Music.pdm to test them in a composition,

before converting them to .sam.  If you create instrument files

with "sustained" (looped) regions, Snd2sam will preserve the

looping information for you - so you can make your own looped

.mod samples.

Snd2sam can require a lot of memory; it reads the entire .snd

file into RAM.  Generally, you should have 350kB or more

available.  (Since Sound.pdm requires that .snd files fit into

its memory space, and since DeskMate and Sound.pdm require a

lot more memory than Snd2sam, I don't think this is a major

limitation.)

This program converts a DeskMate Sound.pdm instrument or sound

file to one or more .sam files for use with Amiga .mod editors,

particularly ModEdit v3.01.  The input .snd file must be

uncompressed.  For each note in the instrument file, two output

files are created:  (1) a .sam file, and (2) a .not (ASCII)

file giving needed information about the .sam file, including

its pitch, large-scale tuning, transposition, and looping

parameters.  Since the .sam file is a headerless format, the

user is required to enter the information from the .not file

manually when using the sample in a .mod editor.

The syntax is:

SND2SAM <.snd file> [<directory>]

The input .snd filename is required and may include drive and

path.  If no extension is specified, it defaults to .snd; a

file without an extension can by used by ending the name with a

period.

The second parameter, which is optional, is the directory where

the output .sam and .not files will be placed.  If not

specified, it defaults to the current directory.

The output filenames are generated from the input .snd

filename.  For the first note, the filename of the .snd file

(without drive, path, or extension) is taken, and extensions of

.sam and .not are attached.  For the second and subsequent

notes, a digit 2-9 or letter A-G is appended to the filename,

overwriting the last character of the filename if necessary.

For example, if the following is entered (where piano.snd is an

instrument file with 3 notes defined):

snd2sam a:piano

The following files will be created in the current directory:

piano.sam

piano.not

piano2.sam

piano2.not

piano3.sam

piano3.not

For a note in an instrument file with pitch and sustain set,

the .not file will take the following format.  In this example,

clarinet.snd has at least one note defined, and note 1 is C3 in

Sound.pdm (middle C, or C2 in .mod pitch).

Data for sample file clarinet.sam

Actual pitch at C2:  G1 finetune +1

Tuning for ModEdit v3.01:

Set tuning to:  G2

Transpose up 1 octave(s).

Tuning for other editors:

Transpose up 5 semitone(s).

Sample is looped.

Repeat start:  3828 (1914 words)

Repeat length:  1356 (678 words)

The .not file begins with the name of the sample file it

describes.  The next line gives the actual pitch of the note

(in .mod pitch) if played back at period 428.  The pitch given

here need not be a valid .mod pitch; it can have an octave

number from -1 to 4.  The finetune is somewhat of a mathemati-

cal artifact; theoretically, given what I know of the Tandy

DAC, the .mod format, and sound frequencies, that is what it

should be.  On my TL, however, I find that a finetune of 0 is

correct.  You will have to experiment to determine the right

finetune for your system, but the finetune you use should be

the same for every .snd file you convert.  Likely values to

specify are -1 and 0.

The pitch is based on the value you specify in Sound.pdm.  It

has to be adjusted for use in a .mod file.  In Sound.pdm, all

instrument files are recorded at 11kHz, and note pitches are

based on that rate.  To make different notes, Music.pdm plays

the note samples back at higher or lower rates, and it bases

the playback rate on the pitch of the note at 11kHz (i.e., the

pitch at which you recorded the note).  This method of playing

different notes, called "frequency shifting," is the same

method used on the Amiga (and PC .mod players).  On the Amiga,

there is no way to specify the implied note; all notes

(samples) are assumed to play at middle C (C2) when played back

at 8363Hz.  If you have samples that play at a different pitch

at that speed, it is your responsibility to transpose the notes

(as if you were scoring for a G or B-flat instrument in an

orchestra) before entering them into the module.  The transpos-

ition information displayed tells you how many half-steps up or

down you must transpose.

ModEdit v3.01 is an exception to the rule.  That program allows

you to set a "large-scale" tuning value for a sample so that

notes you enter for that sample are automatically transposed.

This feature is unique to ModEdit, as far as I know; it is not

part of the .mod format.  If you are using ModEdit, you can use

the large-scale tuning attribute to select a base pitch in

octave 2, and you will then only have to transpose (if at all)

in octaves.

The next few lines in the .not file describe how to set the

large-scale tuning in ModEdit so that the pitch will be true (a

C will be a C, a D-flat a D-flat, etc.) and so that the range

will be as large as possible (a large-scale tuning in octave 2

is always selected).  Depending on the .snd file, it may be

necessary to transpose the sample's notes one or more octaves

in either direction.  In this example, the notes will sound one

octave lower than they are written when the tuning is set as

indicated, so if working from a musical score one must

transpose them up one octave.  Note that you can set the large-

scale tuning to G1 (the pitch given as the "actual pitch") and

not have to transpose at all - but then the range of the

instrument will be limited (see the ModEdit documentation on

sample attributes).

Other .mod editors do not offer large-scale tuning; a note

played at period 428 (8363Hz) is always displayed as C2.  In

this case, the sample's notes will sound 5 semitones lower than

they are written, so if working from a musical score one must

transpose them up 5 semitones.

The last part is the sample's looping information.  In the

example, the first note of clarinet.snd has the sustain region

set, and this region will be used as the repeat region for the

sample.  The repeat start and length are given in bytes (since

ModEdit requires bytes) and in words (which is what the .mod

format requires, and what other .mod editors may use).

If you convert a note with "sustain" set, the part of the note

after the sustain will be discarded.  The sustain section in

the .snd note will become the looped section in the .mod

sample.  Also, since the Tandy allows you to set the sustain

interval to the byte, while the Amiga only lets you set the

looping to the word, the beginning and end of the sustain

interval will be rounded off to a multiple of two.  (The file

size is also rounded off, for the same reason.)

The .sam file is a headerless format; it contains no informa-

tion about large-scale tuning, finetune or looping.  You have

to use the .not file to enter these values manually in your

.mod editor.

For a sound file, or for a note in an instrument file with no

pitch or sustain set, the .not file will take the following

format:

Data for sample file meep.sam

No pitch set

Sample is not looped.

If you didn't set the pitch in the .snd file, Snd2wav can't

help you with tuning, so that information is omitted.

The .sam file will consist of a zero word followed by 8-bit

signed PCM samples.  (The zero word is another Amiga artifact.)

Known bug:  Snd2sam cannot use a read-only .snd file.  This is

a bug in Turbo Pascal's runtime library; it is impossible to

open a read-only binary file (one of the great advantages of

using high-level languages, I suppose ...).  The bug applies

only to files made read-only with the ATTRIB +R command, not to

files on write-protected floppies.  The only way to fix this is

to rewrite TP's file handling routines in assembler; I have not

done so.  Use ATTRIB -R to unprotect the .snd file (Snd2sam

does not modify the .snd file).

When you run Snd2sam, you will see something like this:

I:\>snd2sam piano test

Snd2sam - DeskMate .snd to Amiga .mod sample conversion program

Reading note information ...

Reading sample data ...

Writing to directory "test":

Writing file piano.sam ...

Writing file piano.not ...

Writing file piano2.sam ...

Writing file piano2.not ...

Writing file piano3.sam ...

Writing file piano3.not ...

Done.

It will take only a few seconds on a hard drive.  As always,

things can go wrong, and Snd2sam will provide a message if they

do.  Here's one:

Syntax:

SND2SAM <.snd file> [<output directory>]

You get this message if you invoke Snd2sam without parameters.

The input .snd file name is required.

Unable to open file:

qqq.snd

File qqq.snd does not exist.  You also get this message if you

try to convert a read-only .snd file (see bug note above).

File "modnotes." is not an .snd file.

The specified input file is not a DeskMate .snd file or is

corrupt.  The .snd extension is used by many different systems

for many different types of files.  You should not assume

anything about a file with an .snd extension.

Unexpected end-of-file reading .snd header of file:

qqq.snd

The .snd file is corrupt or truncated, or the specified input

file is not an .snd file and is less than 16 bytes in length.

File "h:piano.snd" is compressed.

Use Sound.pdm to uncompress before converting.

Snd2sam can only convert uncompressed .snd files.  To convert a

file for which you get this message, load the file into

Sound.pdm and select the Compression option from the File menu.

Select "No compression," then Save.

Error reading input file - halting.

This means what it says; probably the disk structure is

corrupt.

Insufficient memory.

There is not enough memory to read the .snd file in.  You are

most likely to get this if you run Snd2sam when shelled out of

something.

Note too long for .mod sample - note skipped.

DeskMate Sound and Music set no limits on how long a note

sample can be (except that the sample must fit in memory).

Amiga .mod samples are limited to a maximum of 128k, and this

message indicates that the sample for the note is longer than

that.  Note that many PC .mod players will not deal correctly

with samples longer than 64k, although Snd2sam will convert

them.  If you get this message, you will have to edit the note

with Sound.pdm to make it shorter.  In general, if you need to

play long notes in your .mod, it is better to create a looped

sample - you avoid problems with the PC's 64k limit, and your

.mod will take less RAM and disk space.  One note is skipped if

you see this message, but if there are other notes in the

instrument file they will be converted.

Note contains no sound data - note skipped.

This message indicates that the file contains a note that was

created but never recorded into.  It is purely informational;

the rest of the notes will be converted.

Error writing output file - halting.

As above, this means what it says.  Run Chkdsk to see if your

disk is messed up.

Disk full - halting.

Self-explanatory.  If part of the .snd has been converted, the

.snd and .not files for that part will be on the disk.

Error creating file "piano.sam" in directory

"d:" - halting.

The output directory specified is not writeable (in this case,

D: is a ROM disk) or is the root directory and is full.

Invalid sample directory:

qqq

The directory you specified for the output files does not

exist.  Note that the second command-line parameter is a

directory, not a filename.

There are a couple other possible error messages.  They

probably indicate that the .snd file is corrupt.

Setdac

------

Setdac sets up the Tandy DAC, either to operate in direct mode

or to enable the joystick.  When set up to work in direct mode,

the DAC acts as if it were a plain homemade DAC (or a Covox

Speech Thing) attached to a parallel port.  A number of

programs that don't directly support the Tandy DAC support dumb

DAC's like that, and Setdac allows the Tandy DAC to be used

with those programs.  With the /J option, Setdac resets the DAC

to enable the joystick when you're done playing sounds.

The syntax is:

SETDAC [/Pn] [/U] [/J] [/I]

If no switches are specified, Setdac sets the DAC for direct

(byte-by-byte) access and displays the hexadecimal port address

that sound programs should be set up to use with it.  I would

guess that for most sound programs that use a parallel-port

DAC, you won't need to specify any switches with Setdac.

The /Pn option, where n is 1, 2 or 3, will set up the DAC in

direct mode, display the port address, and additionally patch

the BIOS parallel port list in low memory to make the DAC look

like one of the system's parallel ports.  For example, my Tandy

1000TL has one built-in printer port.  If I run a system

information utility like MSD, it tells me that I have 1 LPT:

port, LPT1: at port 0378h.  Now I go:

G:\MYPROGS\TSPAK>SETDAC /P2

LPT: port set for DAC.

Tandy DAC at port 00C5h

If I run MSD again, it now tells me that I have 2 LPT: ports:

LPT1: at port 0378h as before, and LPT2: at port 00C5h.  Of

course, it would be perfectly possible for me to go SETDAC /P1

instead.  In that case, MSD would tell me that I have 1 LPT:

port, LPT1: at port 00C5h, and the built-in parallel port would

become invisible (DOS would try to print by sending characters

to the DAC!).  It is fine to make an existing LPT: port

invisible - Setdac will save the old port number (see below),

so when you're done playing sound you can use SETDAC /U to set

the LPT: ports back the way they were.

Why would you want to do this? you might ask.  Well, some

programs that support a DAC on a parallel port won't let you

set the port address (Multiplayer and Modedit do allow it, so

you don't need /Pn with them).  Instead, they try to be user-

friendly and just ask you for the LPT: port number, then go

look up the address for that LPT: port in the table that the

BIOS keeps in low memory.  With a program like that, you would

not be able to set it up directly to use port 00C5h for the

DAC.  Instead, you run SETDAC /Pn and make the DAC be an LPT:

port, then tell the program the LPT: port.  Some programs don't

even ask for the LPT: port number - they just assume that the

DAC is on LPT1: (the Digpak Covox Speech Thing driver is like

that).

One word, though.  Some programs that use LPT: port numbers

rather than I/O ports (i.e., LPT2: rather than 00C5h) have the

port addresses hard-coded.  These programs don't even look at

the BIOS table to get the parallel port address - they just

assume that LPT1:, LPT2:, etc. are at standard addresses.

There is, unfortunately, no way to make such programs work with

the Tandy DAC.  (Well, no *easy* way - you would have to patch

the executable.)

The /U option undoes what /Pn did by setting the BIOS parallel

port list back the way it was.  If as in the example above I

had used SETDAC /P2 to make the DAC be LPT2:, then when I was

finished playing sound I could go SETDAC /U to make LPT2:

disappear.  If I run MSD again after that, it tells me once

more that I have only one LPT: port, LPT1: at 0378h.  If I had

used SETDAC /P1, thus making the printer invisible, SETDAC /U

would make the printer visible again.  You should not do SETDAC

/Pn more than once without doing SETDAC /U, as Setdac will get

confused if there is more than one LPT: port saved.

Setdac is not a TSR, just a normal program; it should be OK to

run it when you are shelled out of something (note, however,

that if you shell out of the program you want to use it with,

that program might have already checked the BIOS parallel port

list).  It is not necessary to type SETDAC /U to unload it when

you are done playing sound.  The only reason for /U is if you

changed the BIOS parallel port list with /Pn and want to change

it back.

When you use /Pn to set the LPTn: address, Setdac saves the old

port number in one of two places in memory:  the master

environment, or the BIOS inter-process communication area.  The

master environment is where COMMAND.COM keeps the environment

variables that you set in AUTOEXEC.BAT.  It is called the

"master" environment because each program gets its own private

copy of the environment.  Say you run your favorite text editor

from the DOS command line, shell out, and run a .gif viewer or

something.  Then there will be 4 environments:  the master

environment, the text editor's environment, the environment of

the secondary COMMAND.COM, and the environment of the .gif

viewer, each being a copy of the one before.  If you shell out

of the editor and try to use the SET command to set a DOS

environment variable, then when you exit out of the editor you

shelled out of, you will see that the SET did not "stick" - the

variable you tried to SET, isn't.  That's because it was set in

the environment of the secondary COMMAND.COM, which is gone now

that you're not shelled out of the text editor any more.  (Type

SET with no parameters at the DOS command line to see what

variables are set.)  Variables that are set in the master

environment, though, do stick, so that is where Setdac saves

the old LPT: port address if it can.  For example, if I used

SETDAC /P1 to make the DAC be LPT1:, Setdac would set the

following variable in the master environment:

_P1=0378

Now if I run SETDAC /U, the program will find this variable and

know how to fix the BIOS parallel port list so that the printer

works again.  It will also remove the variable from the master

environment.

Sometimes Setdac can't use the master environment because

there's not enough room there (like the "Out of Environment

Space" message you get from COMMAND.COM when you try to set too

many variables).  If that happens, the old port address will be

saved in the BIOS inter-process communication area, a 16-byte

space in low memory that the BIOS sets aside for applications

to use.  The IPC is not as safe a place to put stuff as the

master environment, since any program is theoretically free to

put anything it wants there and overwrite what was there

before.  It should generally be OK, though, because few

programs actually use it, preferring instead to use

configuration files and the like to pass parameters around

between modules.

The /I option, if you use it with /Pn, will force Setdac to use

the IPC instead of the master environment.  It is necessary to

use undocumented DOS calls to get at the master environment, so

it's conceivable that saving the old parallel port address

there could cause problems if you're running some weird shell

instead of COMMAND.COM.  You probably don't need /I, but it's

there if you do.  If you use /I with /U, Setdac will skip

looking for the saved LPT: port address in the master

environment and go straight to the IPC.

The /J option sets the DAC to reenable the joystick after

you're done playing sound.  You can also run SETDAC /J by

itself (i.e., without having run Setdac before) if you have a

poorly-written Tandy DAC program that doesn't reset the DAC

when it's done.

Setdac is thus intended to be run before (and possibly after)

another program that supports dumb DAC's.  The best way to use

Setdac is to create a batch file for each program you want to

use it with, for example:

I:\>copy con mt.bat

setdac

multip

setdac /j

^Z

1 File(s) copied

With Setdac.com and Multip.exe in my path, I can now type "MT"

to run MultiPlayer.  Assuming MultiPlayer is configured for the

Tandy chip (see below), I can now use the DAC as an output

device with it.  When MultiPlayer exits, "setdac /j" will reset

the DAC to reenable the joystick.  Similarly, if I had a

program Foobar.exe that did not allow me to set the port

address of the DAC, I could make a batch file for it like this:

I:\>copy con fb.bat

setdac /p2

foobar

setdac /u /j

^Z

1 File(s) copied

I would set up Foobar to use a DAC on LPT2: and type "FB" to

run Foobar.

Setdac returns the following ERRORLEVELs for use in batch

files:

0   if successful

1   if Tandy DAC not found

2   if command-line syntax is invalid

3   if /U failed to restore the LPTn: address

It should be noted that writing sound data byte-by-byte to the

DAC is not the best way to use the chip.  Doing it this way

puts a severe strain on the processor, limiting the sampling

rate and consequently the sound quality you can achieve.  It is

preferable to use the Tandy DAC in DMA mode with a program that

directly supports it.  Setdac is just a hack to enable programs

that otherwise do not support the Tandy chip to use it, albeit

not the way it was meant to be used.

Setdac can display a number of messages.  If it detects a Tandy

DAC in your system, it will display its port address:

Tandy DAC at port 00C5h.

If a Tandy DAC is not found, the following will be displayed:

Tandy DAC not detected.

Setdac exits without doing anything in this case.  If you use

the /Pn option, you get this:

LPT: port set for DAC.

When you use /U to restore the BIOS parallel port list, you

will normally see this:

LPT: port restored.

If you try to do SETDAC /U without having used /Pn before, you

get this message:

Saved LPT: port not found, couldn't restore.

That message can also happen if the old LPTn: port address was

saved in the IPC and some other program mangled it, or if it

was saved in the master environment and that didn't work for

some reason.  You can increase the size of the master

environment by using the SHELL= command in your CONFIG.SYS

file; consult a book on DOS about that.  If you're running some

weird shell and you get that message, try the /I option and see

if it helps.

If you use SETDAC /J, you get this message:

Tandy joystick enabled.

If you use invalid command-line switches, or an invalid

combination of them (ex., SETDAC /P1 /U or SETDAC /J/I), you

get this message:

Syntax:

SETDAC [/Pn] [/U] [/J] [/I]

where:

/Pn    set Tandy DAC for direct mode and

set port LPTn: to be the DAC

/U     restore port LPTn: after previous /Pn

/J     set Tandy DAC to enable joystick

/I     use IPC area with /Pn and /U

See TSPAK.DOC for details.

You're reading TSPAK.DOC now ;-).  Note that the above implies

that SETDAC /H will get you the syntax message, since /H is an

invalid option.

Setdac is only for sound output.  It is possible to set the DAC

to perform sound input byte-by-byte, if anyone comes up with a

reason to do so, but the register programming would be

different (i.e., you can't set the DAC to do both input and

output at the same time).

Each application that you use Setdac with will have to be

configured for the Tandy chip.  How you do this will vary with

the application involved.  Generally, you will want to select

"DAC on LPT1:", "Covox DAC", "Covox Speech Thing" or such like

for the output device and set the "I/O port address" or "base

address" to the port number displayed by Setdac.  If the

program just wants an LPT: port number and won't let you set

the port address, try the /Pn option with Setdac (see above).

As with a homemade DAC, you will also need to set the "sampling

rate" or "output frequency" to something your system will

support.  For the best sound quality, set the rate as high as

possible, but remember that if you set it too high your

computer may lock up or crash.  There is no firm guide on what

you should set the sampling rate to; it depends on the

application and the speed of your machine.  Start with the

lowest sampling rate the application allows and increase it by

increments until the program locks up the machine, then set the

rate to the highest value that works without locking up.  I

have a Tandy 1000TL with an 8MHz 80286.  The highest rate I can

use with MultiPlayer is 6000Hz; with ModEdit, the highest rate

is 8000Hz.

Configure MultiPlayer v2.01 for the Tandy DAC as follows.

NOTE:  DO NOT USE MULTIPLAYER V2.11.  THAT VERSION HAS A BUG IN

THE COVOX DAC DRIVER.  Type <alt>-F to bring up the "File" menu

and select "CLASSIC" for the Playing mode and "Covox Mono" for

the Output device.  Type <esc> to exit the "File" menu and

<alt>-S to bring up the "Setup" menu.  Set the Frequency Mix

Rate to "5000", which is the lowest rate MultiPlayer supports;

you will adjust it later.  Optionally, you may want to set Save

Memory Space for Mods to "NO" (this will speed up playback a

little) and increase the Initial Volume (the default is pretty

soft).  Leave the other stuff alone and hit <esc> to exit the

"Setup" menu.  Type <alt>-D to bring up the "Devices" menu and

set the Covox Left I/O Address to the value displayed by

Setdac.  Leave the other stuff alone and hit <esc> twice to

exit MultiPlayer; your settings will be saved.  Now run Setdac

and restart MultiPlayer.  Hit F3 and play a module.  The

Frequency Rate is displayed in the upper right corner.  Hit the

<up arrow> key to increase the rate.  Eventually, one of three

things will happen.  Either (1) MultiPlayer will knock the rate

back down; (2) the music will get horribly staticky; or (3)

your system will lock up.  You are looking for the highest

_sustainable_ rate.  Some parts of a module are easier to play

than others, and MultiPlayer may be able to achieve higher

rates on the "easy" parts, but you want to find the highest

rate that works for both "easy" and "hard" parts of the module.

This is trial and error.  When you find the rate you want, go

back into the "Setup" menu and set the Frequency Mix Rate to

the highest rate you can use.  Hit <esc> to exit MultiPlayer,

and make a batch file to run Setdac each time you run

MultiPlayer, as described above.

Configure ModEdit v3.01 for the Tandy DAC as follows.  Type C

to bring up the "Configure" menu, then type P to set the

Playback device.  Select "B: D/A converter on LPT1" and hit

<enter>.  Next type A to set Address of LPT ports.  Set the

LPT1: address to the value displayed by Setdac and hit <enter>

three times.  Type M to set the Mixing speed.  Type "5000" at

the prompt and hit <enter>.  This is the lowest mixing speed

ModEdit supports; as above, you will increase the speed later.

Now type S to Save configuration and hit <esc> to exit the

"Configure" menu.  Type F to bring up the "File" menu and X to

Exit ModEdit.  Run Setdac and restart ModEdit.  Use the "File"

menu to load a module (ModEdit comes with one) and use the

"Play" menu to play it.  Now it's trial and error to find the

right rate.  Unlike MultiPlayer, ModEdit does not fail

gracefully (i.e., knocking the rate back down) if you set it

too high.  It just locks up your machine - so when you find a

rate that works, use the "Configure" menu to set the Mixing

speed and Save configuration, then go on looking for a better

rate, keeping your finger poised over the ol' red button.

ModEdit is designed to let you go on editing the module while

it plays, so hit some keys while the module is playing to see

whether your machine will keep up.

It is a hassle to find the right sampling rate for an

application, but you only have to do it once.  If you get

frustrated, take a break and walk in the woods (or cornfields

or desert, whatever you have). :-)

Piano

-----

Piano is a silly little program to play the SN76496-equivalent

circuitry on the IBM PC-Jr or Tandy 1000 or 2500 like a piano.

Unlike the other programs, this one does not require an SL/TL

or later model; this one uses the 3-voice chip.  The following

keys are used:

Key             Note

caps lock       not used

left shift      A2

A               A#2

Z               B2

S               not used

X               C3

D               C#3

C               D3

F               D#3

V               E3

G               not used

B               F3

H               F#3

N               G3

J               G#3

M               A3

K               A#3

,               B3

L               not used

.               C4

;               C#4

/               D4

'               D#4

right shift     E4

enter           not used

`               not used

tab             F4

1               F#4

Q               G4

2               G#4

W               A4

3               A#4

E               B4

4               not used

R               C5

5               C#5

T               D5

6               D#5

Y               E5

7               not used

U               F5

8               F#5

I               G5

9               G#5

O               A5

0               A#5

P               B5

-               not used

[               C6

=               C#6

]               D6

backspace       D#6

\               E6

<esc> exits the program.  In accordance with the sound chip's

capabilities, up to 3 notes can be played at a time (on the

PCjr, 2 notes).  Notes given are American Standard Pitch, Equal

Tempered Scale, or as close as I can get; C4 (period) is middle

C.  High notes are not as close to the true frequency as low

ones.

This program always plays notes at the maximum volume; there is

no volume control.

When the program starts, it displays the keyboard layout:

-- Tandy Piano --

1   2   3       5   6       8   9   0       =  backsp

F#4 G#4 A#4     C#5 D#5     F#5 G#5 A#5     C#6  D#6

tab  Q   W   E   R   T   Y   U   I   O   P   [   ]   \

F4  G4  A4  B4  C5  D5  E5  F5  G5  A5  B5  C6  D6  E6

A       D   F       H   J   K       ;   '

A#2     C#3 D#3     F#3 G#3 A#3     C#4 D#4

shift  Z   X   C   V   B   N   M   ,   .   /  shift

A2    B2  C3  D3  E3  F3  G3  A3  B3  C4  D4  E4

Period is middle C.  Hit <escape> to halt the program.

This program was written on a Tandy 1000TL with a 101-key

keyboard.  If you have another keyboard, the key for the top

note will be in the wrong place.  On the 83-key keyboard, the

top note is the \ key on the bottom row next to the left shift.

On the 84-key keyboard, the top note is the \ key on the top

row between = and backspace.  On the old Tandy keyboard (1000TX

and prior), the top note is the left arrow key.  On the PCjr,

there is no top note (no \ key).

This program requires a PCjr or Tandy 1000 (or later Tandy)

with the Texas Instruments SN76496 3-voice sound chip or

equivalent circuitry.  The program will check to see if you

have the chip.  If not, it will display the following:

This program requires a PC-Jr or Tandy 1000 or 2500 with a 3-voice tone

generator.  It will not run on your machine.