STANDARD MICROSYSTEMS CORPORATION

Microsoft LAN Manager v2.x OS/2 Workstation Driver

For TokenCard EliteSeries LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the Microsoft LAN Manager OS/2

workstation driver for version 2.0, 2.1, or 2.2 from the

SuperDisk to your PC.

o  How to install the Microsoft LAN Manager OS/2 workstation

driver for versions 2.0, 2.1, and 2.2.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

The EZStart default directory is C:\MSLANMAN.OS2. Please make a

backup copy of your SuperDisk before EZStart copies files to the

SuperDisk to create an "option" diskette.

INSTALLING THE DRIVER

Version 2.0 OR 2.2

If you have version 2.0 or 2.2:

1. Run SETUP according to the instructions in the LAN Manager manual.

2. When prompted, select "Other Driver".

3. Insert the "options" diskette you created by using EZStart to

copy the driver. Then select the driver.

4. Proceed with the rest of the procedure in the LAN Manager manual.

Version 2.1

If you have version 2.1:

1. Run the SETUP program according to the instructions in the LAN

Manager manual.

2. When prompted to install the driver, select "Other Driver".

3. Insert the "options" diskette you created by using EZStart to

copy the driver. Then select the driver.

4. Proceed with the instructions in the LAN Manager manual to

generate a PROTOCOL.INI file.

5. After the driver is installed, use a text editor to add the

following line to the [SMC8100] area of the PROTOCOL.INI file:

[SMC8100]

DriverName=SMC8100$

6. Reboot your computer.

This completes the installation procedure.

Refer to the general installation document (NDIS.DOC) for

information on PROTOCOL.INI parameters.

SMC and Standard Microsystems are registered trademarks; and

TokenCard, TokenCard Elite, TokenCard Elite/A, Elite Series,

EliteView, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation.  Other products and company names are

registered trademarks or trademarks of their respective holders.