;----------------------------------------------------------------------------

;Rev	Date	 Name	Description

;----------------------------------------------------------------------------

;R10	05/28/96 AVN	Added Safely Return to DOS if Chip Erase/Program Fail.

;R09	05/28/96 AVN	Added AMD28F010 Support PnP Function.

;R08	04/30/96 AVN	Added DMI Support, And Update Version to 5.2B

;			 A : How To Cognize DMI BIOS

;				1. About System

;			 	   Call System DMI Function 50h For Check

;				   DMI BIOS

;				2. Flash File

;				   Check FC000h, If Continual in 4K Range With

;				   Value 0FFh and FB000h Have Decompress Code

;				   => DMI BIOS.

;			 B : If DMI BIOS When Save BIOS, Should Fill in FC000h

;			     With 0FFh For 4K.

;			 C : When Flash BIOS if

;				   ROM BIOS	File BIOS

;				1. DMI		DMI		: Don't Flash FC000h

;				2. NonDMI	DMI		: Flash FC000h

;				3. DMI		NonDMI		: Flash FC000h

;				4. NonDMI	NonDMI		: Flash FC000h

;R06A	03/28/96 AVN	Fixed AWDFLASH.EXE Backward Compatible and new bios

;			Compatible.

;R07	03/27/96 AVN	Added ATMEL 29C010A Flash ROM, And Change VER -> 5.2

;R06	03/27/96 AVN	Fixed FAR CALL EXTRACT_COMPRESS (EXPAND) Return Fail.

;R05A	03/11/96 AVN	Modify Version 5.0 -> 5.1 For Added INTEL 8K Bootblock

;			Flash Support.

;R04A	02/27/96 AVN	There is Coding Error, And Take out Now First For user.

;R05	02/27/96 AVN	Added Allways Program FE000h-FFFFFh For INTEL 28f001-BXT.

;R04	02/01/96 AVN	Fixed Noncompress (128K), Copy Flash Hook SEG Error.

;R03	10/19/95 AVN	Added MXIC 28F1000P 16K Block PnP Support.

;R02	10/19/95 AVN	For longger BIOS Name String.

;R01	09/19/95 HTR	Fixed system hang when utility running, if NCR 810

;			installed.  Because NCR 810 BIOS will use 3K system

;			RAM, when we decompress system BIOS this will destroy.

;R00	06/01/95 AVN	Rewrite Awdflash.asm