Subject:  Hp48 program

This program for the HP48 calculates the voltages at the nodes of a general

linear circuit by solving the equation :

Y.V = I

where  Y is the (Complex) admittance matrix (I'll define this later), V is a

vector of the (unknown) voltages at the nodes, and I is the vector of current

sources (known). The program will calculate the voltages at a given frequency

or will plot the Amplitude or phase as a function of frequency.

Theory

------

Iij

*-->-----Zij-----*

i                j

Now, Iij = (Vi-Vj)*Yij   (where Yij=1/Zij is the complex admittance)

and also Sum(j, Iij) = Sum(current sources connected to node i) by Kirchoff 1

Therefore, we can write the circuit in matrix form as follows:

for each admittance Yij, add it to the complex matrix Y[] like this :

add Yij to the positions Y[i,i] and Y[j,j] and subtract it from the positions

Y[i,j] and [j,i]. Repeat this for all the components in the circuit. Now create

a column vector of current sources, with a source I from node i to node j being

represented by adding I to the element j and subtracting it from the element i

of the vector. Then we can solve the equation for the V vector as given above.

There is however a problem! The system is _always_ singular, which

corresponds to the physical statement that only potential _differences_ are

measured. However, if we assign node 0 as ground, and only put nodes 1 and

higher in the matrix, then the system has a solution (in general), so we can

procede.

This theory can be found (and much more clearly explained!!) in most books on

AC circuit Theory.

This program converts the inputed components into 3 matrices MatR,MatL,MatC

which represent the Resistive, Inductive and Capacitive components of Y[]. The

routine F\->V calculates Y[] for the given frequency (Y=MatR + 2*pi*i*MatC +

1/(2*pi*i)*MatL) and then solves the equation

Using the program

-----------------

Download the source (given at the end of this article) to your HP48 and select

its directory. Then select the CST custom menu.

An example circuit (the LRC series resonant circuit) is loaded with the

program. To see it, Do NOT CLEAR,  then COMPILE, select output node 3, 1000

10000 FRNG, and press amplitude. Then Wait!

Firstly clear the circuit by pressing CLEAR (2nd Key on the 2nd page of CST)

Then enter your circuit (See below for details)

Then compile your circuit by pressing COMPILE (3rd key on 2nd page of CST)

If you want to see the voltages on the nodes of your circuit at a given

frequency, put the frequency on Top of stack, and press F\->V (4th key on 2nd

page of CST). A vector is returned to the stack giving the voltages of all the

nodes in your circuit.

To plot out either the amplitude or phase response, you must first select the

node that you wish to plot. To do this, press the 5th key on the 2nd page of

CST (labeled with an output icon), and it will prompt you for the node to use.

If you would rather select the node from top of stack, then Left shift the

output key. Then select the frequency range using the FRNG key (2nd key on 3rd

page of CST) which is simply the XRNG function and press either Amplitude (6th

key on the 2nd page) or Phase (1st key on 3rd page) and wait for the graph.

Entering your circuit

---------------------

This program handles 7 types of circuit element which are selected by the first

7 keys on the CST menu. If you would rather take the values from the stack, use

the same keys left shifted. In either case, the nodes are prompted for by the

program.

For each key I'll give the values it expects to get from the stack if left

shifted, or prompts for if simply pressed

(note: you may use unit objects for the values, provided they are correct!)

Resistors - R (Ohms)

Capacitors - C (Farads)

Inductors - L (Henries)

Current source - I(amps)

Voltage source - V(volts), r(Ohms - internal resistance must not be 0)

IcIs (Current Controlled Current Source - e.g. Bipolar Transistor), Beta and

input resistance. The nodes are :

Input to------+      +--------- Output To

!      !

/      O

\      O

/      !

!      !

In from ------+      +----------Output from

VcIs (Voltage controlled Current Source - e.g. FET), gm (Siemens)

Nodes are as for IcIs

Printing out your circuit

-------------------------

To print out the components in your circuit press the 3rd key in the 3rd page

of CST (labeled with a printer Icon). The stack will be left full of strings

representing the elements of your circuit for those users without a printer.

Subroutine \->ENG

-----------------

This routine may be useful in other programs, so I'll document it here:

Input stack     2 : Real number                e.g   2:  10000

1 : "Unit as string:                 1:    "m"

Output          1:  "string of number                1: "'10_km'"

and unit in engineering