W800 (RF Direct) Installation Instructions:

Required equipment:

Palm Pad (or other X10 remote)

W800 adapter (with 9 volt power supply)

You must have Winamp installed on the same computer you are planning to install winX on.

If  Winamp does not reside in your “C:\Program Files\Winamp”, email tech support for further instructions.

Connect the DB9 connector on your W800 adapter to an available serial port on your computer.  Make sure that the 9 volt power connection is also made.

The precise order for the first time startup is as follows:

- Close Winamp, if running.

- Install winX program (setup.exe).

- Start Winamp.

- Start winX.

- Click on “Setup” button.

- This will open a Setup window. In the upper right hand corner, select the same serial port that you have plugged your winX adapter into, and click “apply” button.

- If all goes well, you should get a message “W800 adapter detected”.  A serial link has been established between your PC and the winX unit.

- Confirm the RF  link by pushing any house code - unit code key on your  Palm Pad.  Proper operation is confirmed by viewing the main winX window, lower right hand box, labeled X10 Cmd.  You should see the same HC-UC just transmitted.

- Back at the “Setup” window:

- Select the house code - unit code you wish to use and push “Apply”.

- Assign m3u playlists to the 8 keys on your Palm Pad.  Using the file boxes, locate your playlists.  They will be displayed in the large text box.  Drag and drop any of these playlists in “playlist assignment” boxes #1 through #8.

- Check out operation by pushing the playlist #1 button (topmost - left) on your Palm Pad.  The playlist you just assigned to the #1 box should have been loaded into Winamp.

Congratulations!

- Refer to the Web help tab for additional information and troubleshooting help.