------------------------------------------------------------

GUIDE TO VGA 1024 SOFTWARE DRIVERS

------------------------------------------------------------

Your VGA 1024 video controller (either an adapter or main

logic board circuitry) includes software drivers for running

several popular applications under the controller's Super

VGA screen modes. It also includes INSTALL.EXE, a program

for installing these drivers. This document supplements the

information in INSTALL.EXE.

Press [Page Up] and [Page Down] to view this document a

screen at a time. Press the up and down direction keys to

scroll through the document. To exit the document at any

time, press [Esc].

NOTE: The drivers are supplied by a third-party vendor for

the convenience of advanced users.

Throughout this document, the following conventions are used

to differentiate various types of information:

.    Words enclosed in square brackets ([]) represent keys

on the keyboard. For example: [Enter].

.    Groups of keys are printed like this: [Ctrl+Alt+Del].

Press and hold the keys in the order shown ([Ctrl],

[Alt], and [Del], in this example). Then, release the

keys.

.    Words enclosed in angle brackets (<>) are placeholders

for variable information. For example, the following

command for loading the AutoCAD 16-color, 1024 x 768

driver indicates that you type ds102416 followed by a

variable value:

ds102416 <xx>

.    Filenames, directory names, command names, and command

switches are shown in all uppercase. For example, C:\WP

and WDPVGA1A.WPD. You can enter such information in any

mixture of upper- and lowercase characters.

Guide to VGA 1024 Software Drivers                    Page 2

------------------------------------------------------------

CONTENTS

------------------------------------------------------------

INTRODUCTION ............................................. 3

Super VGA Notes ...... ................................... 7

Before You Use the Software Drivers ...................... 8

VGA1024.EXE and INSTALL.EXE .............................. 8

AUTOCAD DRIVERS .......................................... 9

Using a Batch File to Run AutoCAD ....................... 11

Configuring for the AutoCAD DisplayList Driver .......... 11

Changing the Default AutoCAD Colors ..................... 12

FRAMEWORK II DRIVERS .................................... 14

GENERIC CADD DRIVERS .................................... 16

LOTUS 1-2-3 AND LOTUS SYMPHONY DRIVERS .................. 17

Configuring 1-2-3 or Symphony for the New Drivers ....... 18

Running 1-2-3 or Symphony with the New Driver Set ....... 19

OS/2 DRIVERS ............................................ 21

If You Have Already Installed OS/2 ...................... 21

If You Have Not Already Installed OS/2 .................. 21

Re-installing the Standard VGA Driver ................... 22

VENTURA 1.1 and 2.0 DRIVERS ............................. 23

VERSACAD DRIVERS ........................................ 24

VESA.EXE ................................................ 26

WINDOWS 3 DRIVERS ....................................... 27

Choosing a Configuration ................................ 27

Installing Windows Drivers .............................. 28

Configuring Windows ..................................... 28

WORDSTAR PROFESSIONAL DRIVERS ........................... 30

Installing WordStar Professional for Use in

132-Column Text Modes .............................. 30

Configuring WordStar Professional for Use in

132-Column Text Modes .............................. 31

VGA AND SUPER VGA PROGRAMMING ........................... 33

Standard VGA Modes ...................................... 34

Super VGA Modes ......................................... 36

Guide to VGA 1024 Software Drivers                    Page 3

------------------------------------------------------------

INTRODUCTION

------------------------------------------------------------

The VGA 1024 video controller supports the following Super

VGA modes in addition to the standard VGA modes:

.    1024 x 768 graphics

.    800 x 600 graphics

.    640 x 400, 256-color graphics

.    132-column x 25-line text

.    132-column x 43-line text

To use these Super VGA modes, the controller comes with

software support for several popular applications. The

following table lists these applications and the Super VGA

modes available with each application. It also includes the

monitor type required for each mode. Additional notes on the

Super VGA modes follow the table.

============================================================

Resolution or    Number of   Monitor         Driver

Columns x Rows   Colors      Needed          Filename

============================================================

AUTOCAD 9/10

1024 x 768       16          multi-freq.     DS102416.EXE

1024 x 768       4           multi-freq.     DS1024X4.EXE

800 x 600        16          multi-freq.     DS800X16.EXE

------------------------------------------------------------

AUTOCAD 10 (DISPLAY LIST DRIVERS)

1024 x 768       16          multi-freq.     WDVGADL1.EXE

800 x 600        16          multi-freq.     WDVGADL8.EXE

------------------------------------------------------------

AUTOSHADE

640 x 480        256         any VGA         RD480.EXE

640 x 400        256         any VGA         RD400.EXE

------------------------------------------------------------

Guide to VGA 1024 Software Drivers                    Page 4

============================================================

Resolution or    Number of   Monitor         Driver

Columns x Rows   Colors      Needed          Filename

============================================================

CADVANCE

1024 x 768       4           multi-freq.     GS1024X4.DRV

1024 x 768       16          multi-freq.     GS102416.DRV

800 x 600        16          multi-freq.     GS800.DRV

640 x 480        16          any VGA         GSVGA.DRV

------------------------------------------------------------

FRAMEWORK II

800 x 600        16          multi-freq.     PVGA5.SC

800 x 600        16          multi-freq.     PVGA4.SC

132 x 43

800 x 600        16          multi-freq.     PVGA3.SC

132 x 25

640 x 480        16          multi-freq.     PVGA2.SC

132 x 43

640 x 480        16          multi-freq.     PVGA1.SC

132 x 25

------------------------------------------------------------

GEM/3, VENTURA 1.1, 2.0

1024 x 768       2           multi-freq.     SD_1K_M.VGA

800 x 600        2           multi-freq.     SD_800_M.VGA

800 x 600        16          multi-freq.     SD_800_C.VGA

1024 x 768       16          multi-freq.     SD_1K_C.VGA

------------------------------------------------------------

Guide to VGA 1024 Software Drivers                    Page 5

============================================================

Resolution or    Number of   Monitor         Driver

Columns x Rows   Colors      Needed          Filename

============================================================

GENERIC CADD LEVEL 3

1024 x 768       2           multi-freq.     P1024X2.VGD

1024 x 768       4           multi-freq.     P1020X4.VGD

1024 x 768       16          multi-freq.     P1024X16.VGD

800 x 600        16          multi-freq.     PARA800.VGD

640 x 400        256         any VGA         P640X400.VGD

640 x 480        256         any VGA         P640X480.VGD

------------------------------------------------------------

LOTUS 1-2-3 - VERSIONS 2.0 AND 2.1

800 x 600        16          multi-freq.     SP1_600S.DRV

640 x 480        256         any VGA         SP1_480S.DRV

132 x 43         16          any VGA         SP0_43S.DRV

132 x 25         16          any VGA         SP0_25S.DRV

------------------------------------------------------------

PCAD LEVEL 2

1024 x 768       16          multi-freq.     DPVGA1K.DRV

800 x 600        16          multi-freq.     DWDPVGA.DRV

------------------------------------------------------------

VERSACAD DESIGN - VERSION 5.3

1024 x 768       16          multi-freq.     PVGA1024.EXE

------------------------------------------------------------

VERSACAD DESIGN - VERSION 5.4

1024 x 768       16          multi-freq.     PVGA1024.EXE

640 x 400        256         any VGA         PVGAFF.EXE

640 x 480        256         any VGA         PVGAFF.EXE

------------------------------------------------------------

Guide to VGA 1024 Software Drivers                    Page 6

============================================================

Resolution or    Number of   Monitor         Driver

Columns x Rows   Colors      Needed          Filename

============================================================

WINDOWS 3.0

1024 x 768       2           multi-freq.     BW1024.DRV

1024 x 768       16          multi-freq.     VGA1024.DRV

800 x 600        16          multi-freq.     VGA800.DRV

640 x 480        256         any VGA         PVGA480.DRV

------------------------------------------------------------

WORDPERFECT 5.0

1024 X 768       16          multi-freq.     WDPVGA1B.WDP

800 x 600        16          multi-freq.     WDPVGA1A.WDP

------------------------------------------------------------

WORDSTAR 3.3, WORDSTAR PROFESSIONAL

132 x 43         16          any VGA         43

132 x 25         16          any VGA         25

------------------------------------------------------------

OS/2 1.2

1024 X 768       16          multi-freq.     OS2800.DLL

800 X 600        16          multi-freq.     OS21024.DLL

============================================================

Guide to VGA 1024 Software Drivers                    Page 7

SUPER VGA NOTES

1024 x 768 Graphics

The VGA 1024 controller supports 1024 x 768 graphics in 2,

4, or 16 colors from a palette of more than 262,000 colors.

The 1024 x 768 mode is interlaced and requires the use of an

interlaced monitor.

The 16-color, 1024 x 768 graphics mode requires 512KB of

video memory.

800 x 600 Graphics

The 800 x 600 graphics mode enables the screen to display at

least 56% more information than standard VGA modes. You

might need to use the vertical and horizontal size and

position controls on your monitor to display the entire 800

x 600 graphics mode image without distortion.

256-Color, 640 x 400 and 640 x 480 Graphics

The 256-color, 640 x 480 graphics mode provides five times

the resolution of standard VGA 256-color graphics. This mode

requires 512KB of video memory.

The 256-color, 640 x 400 graphics mode provides four times

the resolution of standard VGA 256-color graphics.

132-Column Text

The 132-column by 25-line text mode displays 65% more text

than a standard 80-column by 15-line text mode. The 132-

column by 43-line text mode displays 184% more text than the

standard mode.

Guide to VGA 1024 Software Drivers                    Page 8

BEFORE YOU USE THE SOFTWARE DRIVERS

.    Make backup copies of the VGA 1024 Installation

Diskette.

.    Be sure that the VGA 1024 video controller is correctly

configured. (If the controller is an adapter, refer to

the adapter installation guide for instructions.)

.    Translate VGA1024.EXE and INSTALL.EXE from English, if

desired. Then, use INSTALL.EXE to copy VGA1024.EXE to

the hard disk. Refer to "VGA1024.EXE and INSTALL.EXE."

NOTE: Translating the files permanently changes the VGA 1024

Installation Diskette, so be sure you are using a backup of

the diskette.

VGA1024.EXE AND INSTALL.EXE

VGA1024.EXE is the utility that enables your VGA 1024 to

emulate specific video standards. INSTALL.EXE is a program

used to copy VGA1024.EXE and the software drivers to the

hard disk. Both files are provided in English, French,

German, Italian, and Spanish. To select a language other

than English, insert the VGA 1024 Installation Diskette into

Drive A and type these commands:

a: [Enter]

translat [Enter]

Copy VGA1024.EXE to the hard disk by typing:

a: [Enter]

install [Enter]

and selecting "VGA 1024 Utilities" and then "VGA1024.EXE."

Once VGA1024.EXE is on the hard disk, you can execute it as

needed. Refer to the video controller documentation in your

computer user's guide or adapter installation guide for more

information.

NOTE: The drivers described in this documentation use a

color VGA configuration. If you are using a monochrome VGA

monitor, use the command VGA1024 COLOR to switch from

monochrome VGA mapping to color mapping before using the

drivers. For more information on monochrome and color

mapping, refer to the video controller documentation in your

computer user's guide or adapter installation guide for more

information.

Guide to VGA 1024 Software Drivers                    Page 9

------------------------------------------------------------

AUTOCAD DRIVERS

------------------------------------------------------------

AutoCAD Releases 9 and 10 can be configured for use with any

of the following graphics drivers:

===========================================================

Driver               Resolution/Colors

===========================================================

DS102416.EXE         1024 x 768, 16 colors

DS1024X4.EXE         1024 x 768, 4 colors

DS800X16.EXE         800 x 600, 16 colors

===========================================================

The drivers are in the form of AutoCAD Device Interface

(ADI) drivers. ADI is a generalized driver format that lets

you load drivers into memory prior to running AutoCAD.

To use these drivers, you must have AutoCAD Release 9 with a

Version 3.1 ADI "generic" driver or Release 10 with a

Version 4.0 ADI "generic" driver.

All the supplied AutoCAD drivers are configured and used in

a similar manner.

The AutoCAD drivers do not support AutoShade.

To install an AutoCAD driver, run INSTALL on the VGA 1024

Installation Diskette to copy the selected driver to the

directory containing AutoCAD:

.    If you have Release 10 and an expanded memory manager,

INSTALL asks whether you want to include the

DisplayList ADI driver as well as the AutoCAD driver.

The DisplayList driver works only with Release 10 and

an EMM. Refer to "Configuring for the AutoCAD

DisplayList Driver" for more information.

.    The INSTALL program also asks whether you want to copy

the SETCOLR.EXE utility. This utility enables you to

change the 16 default AutoCAD colors. Refer to

"Changing the Default AutoCAD Colors" for more

information on using this utility.

Guide to VGA 1024 Software Drivers                    Page 10

To configure AutoCAD for use with a particular driver:

1.   Run the driver file at the MS-DOS prompt.

If you want to run AutoCAD with an inverted black

background rather than the standard white background,

include the i parameter. For example, type:

ds102416 i [Enter]

If your system uses a network card or other device that

uses Interrupt 7A, run AutoCAD with an interrupt vector

other than 7A by typing:

ds102416 <xx> [Enter]

where <xx> is a two-digit hexadecimal vector.

The screen displays a message similar to the following

to indicate that the selected driver has been loaded

into memory:

- AutoCAD ADI Driver -

VGA ! ! ! 1024 X 768

2.   If you are using a mouse or a graphics tablet that

requires a separate driver, be sure that its driver is

loaded at this point.

3.   Execute ACAD to start the AutoCAD program.

4.   From the Main Menu, choose Option 5 to configure

AutoCAD.

5.   From the Configuration Menu, choose Option 3,

"Configure video display."

6.   Choose the option for "ADI display" driver from the

list of available video drivers. If you are using

Release 9, choose "ADI display v3.l." If you are using

Release 10, choose "ADI display v4.0."

7.   When asked for the hexadecimal interrupt code, choose

"7A" or the other interrupt code you selected.

8.   Be sure all other configuration options are set for

your equipment as appropriate for your system.

9.   Exit to the Main Menu and keep your configuration

changes.

AutoCAD now runs in the selected resolution.

Guide to VGA 1024 Software Drivers                    Page 11

USING A BATCH FILE TO RUN AUTOCAD

The commands to load the pointer driver and start AutoCAD

can easily be incorporated into a batch file. For example,

to use the 16-color, 1024 x 768 driver, you might create a

batch file that looks like this:

DS102416  (Loads 16-color, 1024 x 768 ADI driver.)

MOUSE     (Loads mouse driver.)

ACAD      (Starts AutoCAD program.)

CONFIGURING FOR THE AUTOCAD DISPLAYLIST DRIVER

If you install the DisplayList driver for AutoCAD Release

10, you can change various parameters in the DisplayList

driver by running the configuration program CONF.EXE.

If INSTALL can find CONF.EXE, it will run the program for

you. Otherwise, you can run the program at the system

prompt. Type:

conf <driver.exe> <filename> [Enter]

where:

<driver.exe> is the AutoCAD DisplayList driver (WDVGADL8.EXE

for 800 x 600 graphics or WDVGADL1.EXE for 1024 x 768

graphics).

<filename> is the optional new name of the driver. Using the

<filename> parameter enables you to change the parameters in

a new driver file rather than overwriting the original

driver file.

The CONF program asks questions about the driver

installation interrupt vector, screen mode (single or dual),

number of colors, and other driver parameters.

The DisplayList drivers require expanded memory in which to

store the display lists. The drivers cannot run if an EMM is

not loaded.

Guide to VGA 1024 Software Drivers                    Page 12

For the 1024 x 768 driver, type:

wdvgadL1 F<n> [Enter]

where <n> is the number of kilobytes of expanded memory that

the driver should leave free for use by AutoCAD. For

example, to leave 512KB free, type:

wdvgadL1 f512 [Enter]

For the 800 x 600 driver, type:

wdvgadL8 f<n> [Enter]

where <n> is the number of kilobytes to leave free.

CHANGING THE DEFAULT AUTOCAD COLORS

The SETCOLR.EXE program enables you to change the 16 default

AutoCAD colors. This program displays the 16 colors and lets

you change the RGB (red, green, and blue) values for each

color. When you finish defining the desired colors, press

[Enter] to save the RGB values to a file called RGB.TXT.

If you install SETCOLR along with the AutoCAD driver, the

INSTALL program puts SETCOLR in the AutoCAD directory. Run

SETCOLR from this directory. The driver looks for RGB.TXT

there when you start AutoCAD. If the driver cannot find

RGB.TXT, AutoCAD uses the default colors.

Format for RGB.TXT

RGB.TXT is a text file with 16 lines, one line for each

color in 16-color mode. In 4-color mode, only the first four

lines (0 to 3) are defined.

Each line contains three characters separated by spaces. The

characters represent the intensities of red, green, and blue

in that order. Valid intensities are 0, 1, 2, and 3. Each

line ends with a carriage return.

Guide to VGA 1024 Software Drivers                    Page 13

The default values are as follows:

For 4 colors:        For 16 colors:

0 0 0                0 0 0

3 0 0                3 0 1

0 0 3                3 3 0

3 3 3                1 3 1

3 0 0                0 3 3

0 0 3                1 1 2

3 0 0                3 0 3

3 3 3                3 3 3

3 0 0                1 1 1

3 0 0                3 0 0

3 0 0                2 1 0

3 3 3                0 2 0

3 0 0                1 2 2

0 0 3                0 0 3

3 0 0                2 0 2

3 3 3                2 2 2

NOTE: If you have more than four distinct colors set for the

4-color driver, the driver will reject the RGB.TXT file and

use the default colors.

Guide to VGA 1024 Software Drivers                    Page 14

------------------------------------------------------------

FRAMEWORK II DRIVERS

------------------------------------------------------------

Depending on your choice of screen formats for the "desktop"

and "zoom" screens, you can use Framework II drivers for the

following configurations in addition to the standard IBM

EGA, VGA, and PS/2 configurations:

===========================================================

"Desktop" Format     "Zoom" [F9] Format         Driver

===========================================================

132 x 25 text        640 x 480 graphics       PVGA1.SC

132 x 43 text        640 x 480 graphics       PVGA2.SC

132 x 25 text        800 x 600 graphics       PVGA3.SC

132 x 43 text        800 x 600 graphics       PVGA4.SC

800 x 600 graphics   800 x 600 graphics       PVGA5.SC

===========================================================

To install a Framework II graphics driver:

1.   Install Framework II with a screen driver chosen from

the standard IBM Enhanced Graphics Adapter, VGA, and

PS/2 display drivers according to the instructions in

the Framework II "Getting Started" manual.

2.   Be sure Framework II is functioning properly.

3.   Run the INSTALL program on the VGA 1024 Installation

Diskette. INSTALL copies the selected driver into your

Framework II directory.

Note that the INSTALL program prompts you to format a

diskette to use later. You will use this diskette in

the following procedure.

To configure Framework II for use with a particular driver:

1.   Run the Framework II Setup program on the Framework II

Setup diskette or from your Framework II directory.

Type:

setup [Enter]

2.   At the "Welcome to Framework II's 'Setup' program"

menu, choose Option 2, "All other uses of the setup

program."

Guide to VGA 1024 Software Drivers                    Page 15

3.   Tell the Setup program the location of the FWSETUP

file, which stores the current configuration.

If you are using a diskette-based system, insert System

Disk 2 into Drive B and press [1].

If you are using a hard disk system, choose Option 2

because FWSETUP should be in the current default

directory.

4.   At the Main Menu, choose Option 2 to change individual

configuration settings.

5.   From the Change Configuration Menu, choose 1 to change

primary hardware configurations.

6.   Choose 1 to change the screen driver. Then, choose

Option 7, "I want to enter my own driver file name."

Type the name of the driver you wish to use. (Refer to

the table shown earlier in this section.) Then, press

[Enter].

For example, to choose the driver for 132 x 43 text

mode for the "desktop" and 800 x 600 graphics mode for

the "zoom" function, type:

pvga4.sc [Enter]

Be sure to type the complete driver name, including the

.SC extension.

7.   Type M to return to the Main Menu.

8.   At the Main Menu, type 7 to choose "Save All New

Settings."

9.   When prompted to insert the Setup Disk into Drive A,

insert the diskette you formatted when you installed

the video driver. Then, press the space bar.

Again the Setup program asks where to store the FWSETUP

configuration file.

10.  Choose the option for your setup (1: for diskette-based

systems; 2: for hard disk systems). Then, press a key.

Now, when you start Framework II in the normal manner, it

uses the VGA 1024 capabilities you selected.

Guide to VGA 1024 Software Drivers                    Page 16

------------------------------------------------------------

GENERIC CADD DRIVERS

------------------------------------------------------------

Generic CADD Level 3 can be configured for use with any of

the following graphics drivers:

============================================================

Driver         Resolution/Colors

============================================================

P1024X16.VGD   1024 x 768, 16 colors

P1024X4.VGD    1024 x 768, 4 colors

P1024X2.VGD    1024 x 768, 2 colors (monochrome)

PARA800.VGD    800 x 600, 16 colors

P640X480.VGD   640 x 480, 256 colors

P640X400.VGD   640 x 400, 256 colors

============================================================

To install a Generic CADD graphics driver, run the INSTALL

program on the VGA 1024 Installation Diskette. INSTALL

copies the selected driver to the directory containing

Generic CADD.

If INSTALL can find CONFIG.EXE, it runs this program so that

you can configure the Generic CADD for use with the selected

driver.

If INSTALL cannot find CONFIG.EXE, you must run CONFIG.EXE

manually from the system prompt.

To configure Generic CADD for use with a particular driver:

1.   When CONFIG.EXE displays the current configuration,

press any key as prompted.

2.   Select "1" to select a Video Graphics Display.

3.   Choose the appropriate graphics driver.

4.   Exit to MS-DOS by selecting "8."

5.   Respond "Y" to save the selected driver.

You have completed the installation of Generic CADD Level 3

for the appropriate Super VGA mode. You can start Generic

CADD Level 3 in the normal manner, and it will use the

selected video mode.

Guide to VGA 1024 Software Drivers                    Page 17

------------------------------------------------------------

LOTUS 1-2-3 AND LOTUS SYMPHONY DRIVERS

------------------------------------------------------------

Lotus 1-2-3 (Releases 2.0 and 2.1) and Symphony (Releases

1.1, 1.2, and 2.0) can be configured for use with any of the

following VGA 1024 drivers:

============================================================

Driver               Resolution/Colors or Columns x Rows

============================================================

SP1_600S.DRV         800 x 600

SP1_480S.DRV         640 x 480

SP0_25S.DRV          132-column x 25-line text

SP0_43S.DRV          132-column x 43-line text

============================================================

Using the Lotus INSTALL program, you can combine these

drivers with standard Lotus drivers (80-column x 25-line

text and 640 x 350 graphics) to best suit your needs.

NOTE: The VGA 1024 drivers do not work in Lotus View or

Tutorial. You must use the standard drivers supplied with

your Lotus product to use these programs.

Before you begin, install your Lotus software for use with

the IBM System/2 Model 50/60 display. Refer to the 1-2-3 or

Symphony "Getting Started" manual for instructions.

To install the Lotus/Symphony video drivers on the hard

disk, run the INSTALL program on the VGA 1024 Installation

Diskette. Specify that the INSTALL program should copy the

drivers into your 1-2-3 or Symphony subdirectory.

If the Lotus program is correctly installed, INSTALL:

.    copies the Lotus video drivers to your Lotus

subdirectory

.    deletes any existing file named SINGLE.LBR

.    runs the Lotus INSTALL program, which creates a new

SINGLE.LBR file and lets you configure the Lotus

software for the new drivers

Guide to VGA 1024 Software Drivers                    Page 18

CONFIGURING 1-2-3 OR SYMPHONY FOR THE NEW DRIVERS

Once the driver installation program starts the Lotus

INSTALL program, follow these steps to configure Lotus or

Symphony for the new drivers:

1.   Follow the on-screen instructions to get to the Main

Menu, and choose the "Advanced Options" selection.

2.   At the Advanced Options Menu, choose "Add New Drivers

to Library." Follow the on-screen instructions.

After the INSTALL program adds the new drivers to its

library, it returns you to the Advanced Options Menu.

3.   To select the drivers you want to use, choose "Modify

Current Driver Set" from the Advanced Options Menu.

The INSTALL program presents you with a list of driver

types.

4.   Choose "Text Display." Read the following notes, and

then choose the appropriate text display driver for

your configuration from the list of text drivers:

=======================================================

Text Display Driver                Columns x Rows

=======================================================

Universal Text Display             80 x 25

IBM Enhanced Graphics              80 x 25

IBM System/2 Model 50/60           80 x 25

Enhanced VGA (132 x 25)            132 x 25

Enhanced VGA (132 x 43)            132 x 43

=======================================================

Notes:

.    A text display driver's purpose is to display text for

spreadsheets and documents. The drivers included with

the VGA 1024 video controller let you display text in

several new formats. In addition to the standard VGA 80

x 25 text format, the VGA 1024 video controller can

display a 132 x 25 format or a 132 x 43 format.

.    Lotus drivers support different configuration modes as

well as different resolutions and formats. The supplied

drivers support only separate, or toggle, mode. Toggle

mode allows text and graphs to be displayed on the same

monitor but not at the same time.

Guide to VGA 1024 Software Drivers                    Page 19

5.   Choose "Graph Display." Read the following notes, and

then choose the appropriate graphics display driver for

your configuration from the list:

=======================================================

Graphics Display Drivers           Resolution

=======================================================

Enhanced VGA 800 x 600             800 x 600

Enhanced VGA 640 x 480             640 x 480

IBM Enhanced Graphics              640 x 350

IBM System/2 Model 50/60           640 x 480

=======================================================

Notes:

.    The graphics display driver's purpose is to display

Lotus graphs on the monitor. The drivers included with

the VGA 1024 video controller enable you to display

graphics in resolutions up to 800 x 600.

.    Lotus drivers support different configuration modes as

well as different resolutions and formats. The supplied

drivers support only separate, or toggle, mode. Toggle

mode allows text and graphics to be displayed on the

same monitor but not at the same time. Choose the

"separate" graphics drivers to work in toggle mode.

6.   To return to the Advanced Options Menu, select "Return

to Menu" from the Current Drivers Menu.

7.   Select "Save Changes." INSTALL asks you to verify the

driver set that you wish to save. Enter the name of the

driver set if it is different from the default.

RUNNING 1-2-3 OR SYMPHONY WITH THE NEW DRIVER SET

If you saved your driver set as 123.SET for 1-2-3 or as

LOTUS.SET for Symphony, it is used automatically when you

next start your Lotus software.

NOTE: You can make many different driver sets with different

names for different configurations. When you start 1-2-3 or

Symphony, you can specify which driver set you want to use

by including the driver set filename after the program name.

Guide to VGA 1024 Software Drivers                    Page 20

For example, assume you have saved the following drivers as

800.SET:

.    Universal Text Display (Text)

.    Enhanced VGA 800 x 600 - Separate (Graphics)

and the following drivers as 132COL.SET:

.    Enhanced VGA 132x25 - Separate (Text)

.    Enhanced VGA 640x480 - Separate (Graphics)

You can now use the following command to start 1-2-3 with

the first driver set:

123  800 [Enter]

or the following command to start 1-2-3 with the second

driver set:

123  132col [Enter]

After you install the drivers, run Symphony or 1-2-3 to

verify that the drivers display the text and graphics

correctly.

Guide to VGA 1024 Software Drivers                    Page 21

------------------------------------------------------------

OS/2 DRIVERS

------------------------------------------------------------

Follow the appropriate procedure below to configure OS/2

Version 1.2.

IF YOU HAVE ALREADY INSTALLED OS/2

1.   Start your computer under OS/2.

2.   Enter full-screen OS/2 prompt mode.

3.   Run the OS/2 utility DDINSTAL.EXE.

4.   When prompted for your Device Support Diskette, insert

the VGA 1024 Installation Diskette and press [Enter].

5.   Use the space bar and the [Enter] key to select the

desired resolution.

NOTE: Select only one resolution.

6.   When prompted, insert the OS/2 Installation Diskette

and press RESET to restart the system.

7.   Remove the diskette and press RESET again when

prompted.

OS/2 Presentation Manager now runs in the selected graphics

mode.

IF YOU HAVE NOT ALREADY INSTALLED OS/2

1.   Begin installing OS/2.

2.   When asked whether you have a Device Support Supplement

diskette, answer Yes. Insert the copy of the VGA 1024

Installation Diskette.

3.   Use the space bar and the [Enter] key to select the

desired resolution.

NOTE: Select only one resolution.

4.   Finish installing OS/2 and press RESET when instructed

to restart the system.

OS/2 Presentation Manager now runs in the selected graphics

mode.

Guide to VGA 1024 Software Drivers                    Page 22

RE-INSTALLING THE STANDARD VGA DRIVER

1.   Restart the computer with the OS/2 Installation

Diskette in Drive A.

2.   Press [Esc] to display the system prompt.

3.   Go to the C:\OS2 subdirectory.

4.   Insert OS/2 Disk 5 into Drive A and type the following

commands:

unpack a:\ibmvga.dL@ c:\os2\dLL [Enter]

cd dLL [Enter]

copy ibmvga.dLL display.dLL [Enter]

5.   Restart the computer.

Guide to VGA 1024 Software Drivers                    Page 23

------------------------------------------------------------

VENTURA 1.1 AND 2.0 DRIVERS

------------------------------------------------------------

Ventura Publisher Release 1.1 or 2.0 can be configured to

use any of the following drivers:

============================================================

Driver                Resolution/Colors

============================================================

SD_1K_C.VGA          1024 x 768, 16 colors

SD_1K_M.VGA          1024 x 768, 2 colors (monochrome)

SD_800_C.VGA         800 x 600, 16 colors

SD_800_M.VGA         800 x 600, 2 colors (monochrome)

============================================================

1.   Install Ventura Publisher for "IBM Enhanced card /

Enhanced Display (640x350)" or "IBM Personal System 2

Model 50, 60, 70, 80 VGA, (640x480) two colors"

according to the normal setup instructions included

with Ventura Publisher.

2.   Run the INSTALL program on the VGA 1024 Installation

Diskette, and follow the instructions on the screen.

The INSTALL program copies the selected driver to the

Ventura subdirectory.

To configure Ventura for a particular driver, edit the /S

option in the VP.BAT file to identify the driver you want to

use:

For this driver:     Use this /S option:

SD_1K_C.VGA          /S=SC_1K_C.VGA

SD_1K_M.VGA          /S=SD_1K_M.VGA

SD_800_C.VGA         /S=SD_800_C.VGA

SD_800_M.VGA         /S=SD_800_M.VGA

Now, when you run Ventura Publisher, it operates in the

chosen graphics mode on your multi-frequency monitor.

Guide to VGA 1024 Software Drivers                    Page 24

------------------------------------------------------------

VERSACAD DRIVERS

------------------------------------------------------------

Version 5.3 of VersaCAD DESIGN can be configured for use

with either of the following graphics drivers:

============================================================

Driver                Resolution/Colors

============================================================

PVGA1024.EXE          1024 x 768, 16-color

PVGA.EXE              800 x 600, 16-color

============================================================

Version 5.4 of VersaCAD DESIGN can be configured for use

with any of the following graphics drivers:

============================================================

Driver                Resolution/Colors

============================================================

PVGA1024.EXE          1024 x 768, 16 colors

PVGA.EXE              800 x 600, 16 colors

PVGAFF.EXE            640 x 480, 256 colors

============================================================

To install a VersaCAD driver, run the INSTALL program on the

VGA 1024 Installation Diskette. INSTALL copies the selected

driver to the directory containing VersaCAD.

If INSTALL can find ENVIRO.EXE, it runs this program to

configure VersaCAD for use with the selected driver.

If INSTALL cannot find ENVIRO.EXE, you must run ENVIRO.EXE

manually from the system prompt.

Guide to VGA 1024 Software Drivers                    Page 25

To configure VersaCAD for use with a particular driver:

1.   At Environment Menu 3, select the driver for which you

want to configure the application.

2.   To run the application, enter the appropriate command

at the system prompt.

To run Version 5.3 in:           Enter:

1024 x 768, 16-color mode        vcad53 1024

800 x 600, 16-color mode         vcad53 800

To run Version 5.4 in:           Enter:

1024 x 768, 16-color mode        vcad54 1024

800 x 600, 16-color mode         vcad54 800

640 x 480, 256-color mode        vcad54 256

640 x 400, 256-color mode        vcad54 256

Guide to VGA 1024 Software Drivers                    Page 26

------------------------------------------------------------

VESA.EXE

------------------------------------------------------------

VESA.EXE is a terminate-and-stay-resident (TSR) program that

makes the VGA 1024 video controller BIOS compatible with the

Version 1.0 of the VESA (Video Electronics Standards

Association) Super VGA BIOS Extension Specification.

VESA.EXE provides support for the VESA BIOS calls.

Decompress VESA.EXE by running the INSTALL program on the

VGA 1024 Installation Diskette and selecting "VGA 1024

Utilities." To use VESA.EXE, enter VESA at the system prompt

before you run an application that uses a VESA-compatible

driver.

The following VESA modes are supported by VESA.EXE:

===========================================================

15-Bit         7-Bit           Resolution       Number of

Mode Number    Mode Number                      Colors

===========================================================

100h            --          640 x 400        256

101h            --          640 x 480        256

102h            6AH         800 x 600        16

104h            --          1024 x 768       16

===========================================================

For more information on the VESA specification, call the

Video Electronics Standards Association, (408) 971-7525.

Guide to VGA 1024 Software Drivers                    Page 27

------------------------------------------------------------

WINDOWS 3 DRIVERS

------------------------------------------------------------

Microsoft Windows Version 3 can be configured for use with

any of the following video drivers:

===========================================================

Driver               Resolution/Colors

===========================================================

VGA1024.DRV          1024 x 768, 16 colors

BW1024.DRV           1024 x 768, 2 colors (monochrome)

VGA800.DRV           800 x 600, 16 colors

PVGA480.DRV          640 x 480, 256 colors

===========================================================

The Windows drivers require Microsoft Windows Version 3.

Both the standard Windows product and the "runtime" version

work with these drivers. Applications such as Aldus

PageMaker that run under Windows 3 can be used with these

drivers either using the standard Windows environment or the

"runtime" version of Windows 3.

CHOOSING A CONFIGURATION

Standard Windows applications have complete access to the

256 colors where they can use them. Some applications will

be able to use pure colors rather than complicated color

patterns and will operate more efficiently. Other Windows

applications might use fewer than 256 colors although the

colors are available.

The Windows drivers have been optimized for performance in

all modes. The number of colors on the screen affects the

drawing speed of graphics and text. Fewer colors and lower

resolution result in faster operation.

Choose the resolution and the number of colors based on your

own requirements. For example, for black-and-white desktop

publishing, you might choose 1024 x 768 resolution with only

two colors for the maximum displayed information and faster

drawing speed. For color graphics, you might choose a

configuration with 16 or 256 colors.

Guide to VGA 1024 Software Drivers                    Page 28

INSTALLING WINDOWS DRIVERS

To install the Windows drivers, run the INSTALL program on

the VGA 1024 Installation Diskette.

.    The INSTALL program prompts you for the directory in

which to place the OEM Setup Disk files. Specify a

temporary directory, such as C:\OEMDISK, and remember

the name you specify.

CAUTION: Do not specify C:\WINDOWS, or the directory

where Windows is installed.

CONFIGURING WINDOWS

To configure Windows for use with a particular driver:

1.   If you have not yet installed Windows 3.0 on your

computer, install it by running the Setup program on

Windows Disk 1. To do this, insert the Windows Setup

Disk into Drive A. Switch to Drive A by typing:

a: [Enter]

And start the Setup program by typing:

setup [Enter]

Follow the instructions on the screen to install

Windows.

2.   Go to the directory where Windows is installed. Run

Setup again.

3.   Use the direction keys to move to the Display: line in

the System Information list. The current display type

is highlighted. Press [Enter].

Setup provides a list of display types.

4.   Use the direction keys to scroll to "Other." Press

[Enter].

Setup prompts you for the "manufacturer's display

driver diskette," the disk containing the OEM Setup

files. It displays the default location for these files

as a:\.

Guide to VGA 1024 Software Drivers                    Page 29

5.   The OEM Setup files on the VGA 1024 Install Diskette

are in a compressed format, so you cannot use them.

Instead, tell Setup where to find the decompressed OEM

Setup files on the hard disk. Press the [Backspace] key

to move the cursor backward. In place of a:\, type the

name of the hard disk directory containing the OEM

Setup files (for example, C:\OEMDISK).

6.   Select the driver you wish to use, for example "VGA

1024 640x480 256-color."

Setup returns you to the configuration menu. You are

finished changing the Display: parameter.

7.   Change any other configuration parameters as needed.

(Refer to your Windows documentation.) Then, select

"Accept the configuration shown above."

When the Setup program finishes executing, the selected

driver is installed.

Guide to VGA 1024 Software Drivers                    Page 30

------------------------------------------------------------

WORDSTAR PROFESSIONAL DRIVERS

------------------------------------------------------------

WordStar Professional Releases 4.0 and 5.0 can make use of

132-column text modes without special modification to the

software. Simply tell the software the screen size (in rows

and columns), and the software will take advantage of the

new configuration.

INSTALLING WORDSTAR PROFESSIONAL FOR USE IN

132-COLUMN TEXT MODES

Install WordStar Professional for the appropriate screen

size as described in Appendix C, "Customizing WordStar," of

the WordStar Professional documentation.

1.   Copy WordStar Professional Release 4.0 or 5.0 to your

working diskette or hard disk as directed in your

WordStar Professional documentation.

2.   Start WordStar's WSCHANGE installation program.

3.   Enter the name of your WordStar program file, for

example, WS.EXE.

The installation program prompts you for the file to

which you wish to save your changes.

4.   Enter WS132.EXE as the name.

5.   At the Main Installation Menu, choose Option A for

Console options.

6.   At the Console Menu, choose Option A for Monitor

options.

7.   At the Monitor Menu, choose Option C for Screen sizing.

8.   At the Screen Sizing Menu, choose Option A for Height.

The program asks you to enter the correct value for the

screen format you will use.

9.   Enter 25 for a 132 x 25 format or 43 for a 132 x 43

format.

The program returns you to the Screen Sizing Menu.

Guide to VGA 1024 Software Drivers                    Page 31

10.  Choose Option B for Width.

The program asks you to enter the correct value for the

screen format you will use.

11.  Type 132.

You are returned to the Screen Sizing Menu.

12.  When you have made your choices, type x to return to

the Monitor Menu.

13.  Type x to return to the Console Menu.

14.  Type x to return to the Main Installation Menu.

15.  At the Main Installation Menu, type x to indicate that

you have completed your installation.

The installation program asks you to confirm your

choices. Then, it modifies and saves the WordStar

program.

You will not need to run this procedure again unless you

wish to change the configuration of your WordStar program.

CONFIGURING WORDSTAR PROFESSIONAL FOR USE IN 132-COLUMN TEXT

MODES

Run the INSTALL program on the VGA 1024 Installation

Diskette, and select WordStar Professional. The INSTALL

program:

.    copies VGA1024.EXE into the WordStar directory

.    copies RWS.EXE into the WordStar directory if you are

using WordStar Professional 5.0 or later

.    creates RWS132.BAT for running WordStar in the selected

resolution

Guide to VGA 1024 Software Drivers                    Page 32

The RWS.EXE program is needed to enable WordStar to switch

between 132-column text mode and the graphics mode used for

the page preview function for WordStar 5.0 or later. The

RWS132.BAT file loads the RWS.EXE program for you.

To run WordStar in 132-column mode after installing WordStar

into the WS132.EXE file, type:

rws132 <filename> [Enter]

Guide to VGA 1024 Software Drivers                    Page 33

------------------------------------------------------------

VGA AND SUPER VGA PROGRAMMING

------------------------------------------------------------

This section describes how to access the Super VGA modes of

the VGA video controller. This information is intended for

users familiar with assembly-language programming. An

understanding of this information is not necessary for

normal operation of the video controller.

The VGA standard supports the video modes listed in the

following table. These modes can be accessed through

standard video BIOS calls from assembly language as well as

high-level language routines. When you start in MS-DOS, the

computer is usually in standard, 80-column text

(alphanumeric) mode. On a color system, this is Mode 3+. VGA

640 x 480, 16-color graphics mode is 12H.

The VGA 1024 video controller supports the following Super

VGA modes in addition to the standard VGA modes:

.    1024 x 768 graphics

.    800 x 600 graphics

.    640 x 400, 256-color graphics

.    132-column x 25-line text

.    132-column x 43-line text

Identification numbers for the Super VGA modes are shown in

"SUPER VGA MODES."

Guide to VGA 1024 Software Drivers                    Page 34

STANDARD VGA MODES

============================================================

Mode  Type   Colors   Columns  Buffer Char.   Resolution (3)

(hex)        (1)      x Rows          Size (2)

============================================================

0     text   16/256K  40x25    B8000  8x8     320x200

0*    text   16/256K  40x25    B8000  8x14    320x350

0+    text   16/256K  40x25    B8000  9x16    360x400

------------------------------------------------------------

1     text   16/256K  40x25    B8000  8x8     320x200

1*    text   16/256K  40x25    B8000  8x14    320x350

1+    text   16/256K  40x25    B8000  9x16    360x400

------------------------------------------------------------

2     text   16/256K  80x25    B8000  8x8     640x200

2*    text   16/256K  80x25    B8000  8x14    640x350

2+    text   16/256K  80x25    B8000  9x16    720x400

------------------------------------------------------------

3     text   16/256K  80x25    B8000  8x8     640x200

3*    text   16/256K  80x25    B8000  8x14    640x350

3+    text   16/256K  80x25    B8000  9x16    720x400

------------------------------------------------------------

4     graph. 4        40x25    B8000  8x8     320x200

------------------------------------------------------------

5     graph. 4        40x25    B8000  8x8     320x200

------------------------------------------------------------

6     graph. 2/256K   80x25    B8000  8x8     640x200

------------------------------------------------------------

7     text   4        80x25    B0000  9x14    720x350

7+    text   4        80x25    B0000  9x16    720x400

------------------------------------------------------------

D     graph. 16/256K  40x25    A0000  8x8     320x200

------------------------------------------------------------

E     graph. 16/256K  80x25    A0000  8x8     640x200

------------------------------------------------------------

F     graph. 4        80x25    A0000  8x14    640x350

------------------------------------------------------------

10    graph. 16/256K  80x25    A0000  8x14    640x350

------------------------------------------------------------

11    graph. 2/256K   80x30    A0000  8x16    640x480

------------------------------------------------------------

12    graph. 16/256K  80x30    A0000  8x16    640x480

------------------------------------------------------------

13    graph. 256/256K 40x25    A0000  8x8     320x200

============================================================

(1)  Colors: Where two numbers are given, the first is the

number of colors available at one time and the second

is the number of possible colors. For monochrome modes,

this number refers to the number of attributes.

Guide to VGA 1024 Software Drivers                    Page 35

(2)  Character Size: The size of the matrix that contains

each text character.

(3)  Resolution:  All 200-line modes are "double-scanned" to

display 400 lines on the screen.

*    EGA-style text modes with 8 x 14 and 9 x 14 character

sizes and 350 lines vertical resolution.

+    VGA-style text modes with 9 x 16 character size and 400

lines vertical resolution.

Default modes are 3+ for color monitors and 7+ for

monochrome monitors.

Guide to VGA 1024 Software Drivers                    Page 36

SUPER VGA MODES

============================================================

Mode  Type   Colors   Columns Buffer  Char.  Res.     Equip.

(hex)        (1)      x Rows          Size            Flag

(2)             (3)

============================================================

54*   text   16/256K  132x43   B8000  7x9   924x387   color

54**  text   16/256K  132x43   B8000  8x9   1056x387  color

------------------------------------------------------------

55*   text   16/256K  132x25   B8000  7x16  924x400   color

55**  text   16/256K  132x25   B8000  8x16  1056x400  color

------------------------------------------------------------

56*   text   4        132x43   B0000  7x9   924x387   mono.

56**  text   4        132x43   B0000  8x9   1056x387  mono.

------------------------------------------------------------

57*   text   4        132x25   B0000  7x16  924x400   mono.

57**  text   4        132x25   B0000  8x16  1056x400  mono.

------------------------------------------------------------

58    graph. 16/256K  100x75   A0000  8x8   800x600   color

------------------------------------------------------------

59    graph. 2        100x75   A0000  8x8   800x600   mono.

------------------------------------------------------------

5A    graph. 2        128x48   A0000  8x16  1024x768  mono.

------------------------------------------------------------

5B    graph. 4        128x48   A0000  8x16  1024x768  color

------------------------------------------------------------

5D    graph. 16 (4)   128x48   A0000  8x16  1024x768  color

------------------------------------------------------------

5E    graph. 256/256K 80x30    A0000  8x16  640x400   color

------------------------------------------------------------

5F    graph. 256 (4)  80x25    A0000  8x16  640x480   color

------------------------------------------------------------

6A*** graph. 16/256K  100x75   A0000  8x8   800x600   color

------------------------------------------------------------

6B*** graph. 2        100x75   A0000  8x8   800x600   mono.

============================================================

Guide to VGA 1024 Software Drivers                    Page 37

(1)  Colors: Where two numbers are given, the first is the

number of colors available at one time and the second

is the number of possible colors. For monochrome modes,

this number refers to the number of attributes.

(2)  Character Size: The size of the matrix that contains

each text character.

(3)  Equipment Flag: Required setting of the BIOS data value

at address 40:10 Bits 5 and 4. (0,1 or 1,0 = color; 1,1

= monochrome.)

(4)  Requires 512KB of memory on the VGA 1024 video

controller.

*    Fixed-frequency monitor setting.

**   Multi-frequency monitor setting.

***  VESA modes (different monitor timings from Modes 58 and

59.

If the video controller is an adapter card, the fixed-

frequency or multi-frequency monitor setting is determined

by Switch 1 on the adapter.

Programming the Super VGA modes is similar to programming

the standard VGA video modes of the IBM PS/2 VGA and PS/2

Display Adapter.

Extended BIOS calls are provided to support the VGA 1024

video controller's Super VGA modes. If you are writing to a

driver or an application that uses the Super VGA modes, use

the algorithms and tools described in the file PROGRAM.TXT.

Read or print this file for details on the recommended

methods for programming the VGA 1024 video controller.

Guide to VGA 1024 Software Drivers                    Page 38

January 1991

The information in this document is subject to change

without notice and should not be construed as a commitment

VGA 1024 Drivers and Utilities:

Copyright (C) 1991 Tandy Corporation. All Rights Reserved.

Guide to VGA 1024 Software Drivers:

Copyright (C) 1991 Tandy Corporation. All Rights Reserved.

AutoCAD, AutoShade, and ADI are registered trademarks of

Autodesk, Incorporated.

IBM, PS/2, and Personal System/2 are registered trademarks

of International Business Machines Corporation.

Lotus is a registered trademark of Lotus Development

Corporation.

Excel, MS-DOS, MS OS/2, and Windows are registered

trademarks of Microsoft Corporation.

Paradise is a registered trademark of Western Digital

Corporation.

Ventura Publisher is a registered trademark of Xerox Desktop

Software.

WordStar is a registered trademark of WordStar

International, Inc.

Guide to VGA 1024 Software Drivers                    Page 39

Xerox is a registered trademark of Xerox Corporation.

1-2-3 and Symphony are trademarks of Lotus Development

Corporation.

Cadvance is a trademark of Isicad, Inc.

Framework II is a trademark of Ashton-Tate.

GEM and GEM/3 are trademarks of Digital Research, Inc.

Generic CADD Level 3 is a trademark of Generic Software,

Inc.

VersaCAD is a trademark of VersaCAD Corporation.

WordPerfect is a trademark of WordPerfect Corporation.