%OP%DEIntroduction

%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%CO:A,12,80%%C%%H2%R e l e a s e  v3.00%H2%

%C%Z88 Developers' Notes

%C% v2.00, (C) Copyright Cambridge Computer Ltd. 1989

%C%v2.00 documentation published by Cambridge Computer Ltd. 1989

%C%Authors, v2.00:

%C%Matthew Elton

%C%John Harrison

%C%Consultants, v2.00:

%C%Matthew Soar

%C%Trinity Concepts Ltd

%C%Jim Westwood

%C%Graham French

%C%v3.00: Text ported to PipeDream, re-arranged and improved with additions

%C%Gunther Strube, InterLogic 1995

%P0%

%C%%H2%Disclaimer%H2%

%C%InterLogic will not in any event be liable for any loss, including

%C%consequential loss, caused by any error, defect or inaccuracy in this

%C%information, including but not limited to loss of profit or loss of contracts.

%H2%Release information, 19. September 1995%H2%

This is the third release of the Developers Notes. Many things have been

changed and added since version 2. Further, the new V4 of the operating system

was released long after the V2 notes. Unfortunately, this documentation does

not reflect the bug fixes of V4, nor any improvements in V4. We don't know how

V4 behaves, but hope to aquire information to be updated in these notes as soon

as it gets into our hands. All reported bugs in the system calls reference

refer to the V2.2/3.xx releases of the operating system.

If you find any inaccuracies or inconsistencies, or if you have any comments on

the style and presentation of these documents, then please pass these on in

writing to:

Finally, many thanks to you, Vic, for your support of these notes. Many details

have come to light because of those VERY old system printouts from the birth of

the OZ operating system.

Z88 forever!

Gunther Strube

%H2%Important notice:%H2%

lf you write an application which contains bugs or tries to circumnavigate the

operating system, then it is likely that all the other applications in the Z88

will be affected. The effect may not be immediate, some indiscretions take

weeks or even months to become apparent, but will usually be in the form of a

system crash. You must always remember that the resources of the Z88 are not

devoted exclusively to your application and therefore only use legal

interfaces. Please note that all the internal applications follow all the rules

and use no 'back door' techniques.

%P0%

%H2%Preamble%H2%

The aim of these notes is to provide a comprehensive description of the

hardware and software of the Z88 portable. The machine has a powerful operating

system which provides facilities for software to be added on external cards,

and integrated into the same environment which cares for the built-in programs.

External software can take full advantage of the Z88's facility to switch

between tasks, can use automatically generated command menus with optional help

pages, and is provided with a large number of useful operating system calls, to

access the screen, serial port, internal filing system,printer driver, etc.

%H2%Who these notes are for%H2%

These notes are primarily targeted at the software author interested in writing

applications for the Z88, although it will also be useful to anyone developing

hardware. The emphasis is placed on authors whose programs are destined for

application cards, but a programmer using BASIC and needing to exploit the

facilities of the operating system will find what he or she needs here.

The notes assumes the reader is reasonably familiar with the Z88, and that the

User Guide supplied with it has been read and is available for reference.

Further, some basic knowledge of Z80 assembly language programming has to be

assumed, for while it would be perfectly possible to develop software for the

Z88 using a high level language compiler, no such compiler for use with the Z88

operating system is available. The information here, therefore, is from the

point of view of an application writer developing in assembly language. Of

course, this means that there is more than enough information for custumizing a

compiler if this is the preferred route.

We would anticipate that most developers would use a Z80 assembler running on a

different machine, and would blow EPROM's from that machine to test in the Z88.

Alternatively EPROM's can be blown from the Z88 itself. There is a simple

development environment, which is the BASIC language which comes with the

machine and includes an in-line Z80 assembler. How to use BASIC as a

development environment is detailed in the "BBC BASIC and the Assembler"

section of these notes.

The Z88 Assembler Workbench that is bundled with these notes is a complete

developers package that has all the necessary facilities for programming

applications on the Z88, and contains a modular Z80 assembler, debugger, EPROM

programming software and on-line help, all as resident applications on a 128K

ROM. In addition, a Z80 Cross Assembler is supplied on an IBM formatted

disc to compile Z80 Assembler source files on a PC-compatible computer.

All the assembler examples in these notes have been written in the format used

by the Z80 Assembler supplied in the Z88 Assembler Workbench. Over 50 different

library routines are also supplied, all appropriately commented. Please refer

to 'Wrkbench.doc' for more information on how to use the Z88 workbench

software.

%H2%Structure of these notes%H2%

The notes begins with a general overview of the Z88. It is important to study

this carefully as many important concepts are dealt with, and a good

understanding of this section will make the rest of the notes much easier to

follow. Applications are talked about and guidelines for their construction are

given. Application construction is then covered in detail, with the necessary

static structure needed in the ROM card to let the system know the size of the

card, what applications it contains etc. In this section the data structure for

generating menus, command sequences and help pages is also discussed. Before

moving on to discussing the main body of system calls, and how they are useful

to the application writer, two important subjects are tackled. Firstly the way

in which errors are handled; some errors include a request for an application

to close itself down, and require serious immediate action, whereas others

might simply inform the application that it may have been interrupted - perhaps

the Z88 was switched off for a while. Secondly the memory architecture of the

Z88 and the system memory management routines are explained.

Understanding the memory organisation is the key to successful application

writing, so this section needs to be studied with care.

Following from this material individual calls can be discussed in some detail,

starting with the most basic and commonly used and gravitating towards the more

esoteric. Before the extensive calls reference part of the notes, Z88 hardware

is discussed. The appendices include details of internal application file

formats (ie. PipeDream and Diary), hardware connections and other general

information. A glossary and index is provided.

%P0%

%CO:B,12,68%

%H2%InterLogic%H2%

Att: Gunther Strube

Gl. Kongevej 37, IIth

DK-1610 Kopenhagen V

Denmark

Tlf: (+45) 3131 7570

e-mail gunther@inet.uni-c.dk

%CO:C,12,56%%CO:D,12,44%%CO:E,12,32%

%H2%Rakewell Ltd%H2%

Att: Vic Gerhardi

24 Putnams Drive

Aston Clinton

Aylesbury

Bucks HP22 5HH

Tlf: (+44) 1296 630617

%CO:F,12,20%%CO:G,12,8%