1. INTRO.

World is  a big  place. There  is lot  of happy,  stable-minded

people around. Some of them are using PC/AT-s and they still remain

perfectly sane. However, life  is full of unpredictable  things and

perhaps on one day you decide you need a bigger Hard Disk.

If you have choice  or just a good  chance, then the HD  you've

got, matches the BIOS of  your machine. It means, on  running SETUP

utility program (and this is what  you most likely have to do)  you

can find a  Hard Disk type  entry what matches  perfectly with what

you need. But sometimes not.

It may happen that  you find a HD  type what is close  to right

one,  but  not  close  enough.  Typically, some subtle nuances like

cylinder count,  landing zone  or precompensation  cylinder number,

are different. After  consulting Very Wise  People and TECH!  HELP,

you'll decide to re-program the BIOS.

BIOS programming is a  fascinating hobby, but it  is dangerous.

If you are caring type, then  very soon both your machine and  your

sense of humour  will not work  any more. Also,  some parts of  you

will certainly suffer from UV-radiation. People start to avoid you.

Using HARDFAKE utility,  it is possible  to use different  Hard

Disk Drive(s) without blasting  innocent EPROMS and hating  a world

around. Read on.

2. HOW IT WORKS.

Virtually all data  exchange with disks  goes through the  BIOS

and  (D)OS  routines.  Somewhere  in  this  machinery, the concrete

arguments for Disk Controller (head number, cylinder, write  level,

etc.) will  be figured  out. They  will be  dependent on  Hard Disk

Parameters, typically found in HDP table in BIOS ROM.

On boot  time, the  specific vector  addresses will  be set  to

point the right  entries in HDP  table. HARDFAKE will  generate new

entries with given parameters, put them into memory and correct the

vectors mentioned above. After that, DOS will ignore the parameters

in ROM and use the new ones instead.

3. USAGE.

The simplest way is an example.

HARDFAKE 1 615 8 -1 0 614 17

�  �  �  � �  �  �

�  �  �  � �  �  �� sectors per track;

�  �  �  � �  ����� landing zone;

�  �  �  � �������� offset 8 in param. record (usually 0);

�  �  �  ���������� start of precompensation or -1;

�  �  ������������� number of heads;

�  ���������������� number of cylinders;

������������������� drive number (0 or 1).

If you want to specify  parameters for two HD drives,  just specify

two argument arrays (14 numbers in grand total), like

HARDFAKE 1 615 8 -1 0 614 17 0 733 5 300 0 732 17

If you  like to  experiment, then  you can  call the program as

many times as you like. The previous parameter definitions will  be

forgotten. On each call, only 32  or 48 bytes of data will  be made

resident in memory. If there is wrong number of arguments given  or

if drive number is not 0 or 1, no action will be taken except  help

message printing to the screen.

Typically,  HARDFAKE  call  should  be  placed  in AUTOEXEC.BAT

before calls for any other utilities.

NOTE:  When  reconfiguring  your  system,  it  is  important to

specify correct number of HD drives using the SETUP utility.  Also,

the types specified should be as close to actual ones as  possible.

The NUMBER OF HEADS must  be correct, at least. The  cylinder count

is better to  be specified smaller  but not bigger  than actual. In

other words, the Power On  Self Test must run correctly  and system

must load, before HARDFAKE is able to do something.

4. COPYRIGHT and other conditions.

HARDFAKE is a shareware product. Uncommercial (personal) use is

free  for  anyone.  When  delivering  this  software,  both   files

HARDFAKE.COM  and  HARDFAKE.DOC  (this text)   must  be present and

unmodified. Only bad guys can ask money for a delivery.

Author  of  this  software  (me)  does  not expect any dollars,

roupees or roublees  to be donated.  However, beer (especially  the

Danish Elephant Professional)   and   good  company   are    always

appreciated as a real help  to work on  oncoming  software products

and to support the existing ones.

I don't take any responsibilities for financial, moral or what-

ever losses or damages resulting from use of this software.

If somebody  will find  bugs in this software  or has any other

reasons to  contact me,  it can  be done  via P.O. Box Maximus  BBS

(2:515/881) Tallinn, Estonia.

KalaSoft is my personal venture.

���������������������������������������������������Ŀ

� Villem Alango,  Project Manager                   �

� TARKVARA Innovation Team, Box 3389, Tel: 44 64 12 �

� Tallinn, Estonia                                  �

�����������������������������������������������������