BocaBoard 1004/1008 setup for PC-MOS/386

The printed manual contains an error on page 15 (section 4.2) in

describing the config.sys lines needed to run with PC-MOS/386.

The correct syntax to add to config.sys is as follows:

DEVICE=$SERIAL.SYS~

/AD=0280, IB=64, IN=4, IF=14, OF=16~

/AD=0288, IB=64, IN=4, IF=14, OF=16

DEVICE=C:\PCTERM.SYS

Boca Research UNIX and XENIX driver for BB1004, BB1008, BB2016, BBMC04 and

BBMC16:

Since the distribution is on a DOS floppy, you need to use the

DOS utilities, which are a package of the operating system's Extended

Utilities.  If the DOS utilities are not installed, you need to

install them.

The XENIX driver is compiled for XENIX 386.  A XENIX 286 driver is not

currently available.

The default sio driver that ships with XENIX and some versions

of UNIX is configured by default to search through the system

I/O addresses to identify serial port boards.  When both the sio

driver and the BocaBoard driver are installed, the sio driver may

misidentify the BocaBoard as another serial port board. You can

determine if this is the case by examining the /usr/adm/messages file.

The BocaBoard cannot be simultaneously accessed from the sio driver

and the BocaBoard driver.  For UNIX, the sio driver can be changed

by editing the /etc/conf/pack.d/sio/space.c file (if you are familiar

with C programming) or by using the "configure" utility to remove

the sio driver.  For Xenix, the sio driver is controlled by the

/usr/sys/io/sioconf.c file, which you can change if you are familiar

with C programming and have the Xenix Development System installed.

Alternatively, you can use "configure" to delete the sio driver.

A third alternative is to use the sio driver to drive the BocaBoard.

The 4 and 8 port BocaBoards can be configured to look like

a Hostess board (IO address 140 and IRQ 4 or IO address 200 and

IRQ 3).

BocaBoards have a Port Status Register (PSR) located at offset 7 from

the IO address of the first port.  In other words, if the board is

configured for IO address 0x100, the PSR is located at 0x107.

The 16 port BB2016 and BBMC16 products have two PSR registers,

the first at offset 7 from the first port, the second at offset

71 (0x47 hex).  Essentially, the 16 port boards look like 2

adjacent 8 port boards sharing the same interrupt.

The PSR uses active HIGH logic, meaning that if a port has

interrupted, its corresponding bit in the PSR goes HIGH.

The PSR port is ordered so that the least significant bit of the PSR

is matched to the first (topmost) port on the board.  On the

16 port products, the first port is the closest to the DB37

connector.

Because the BB1004/8 uses a 6-pin RJ-11 connector, some forms of

hardware handshaking are not feasable.  Terminals can use XON/XOFF

and DTR handshaking, but not RTS/CTS.  Modems can only use XON/XOFF

handshaking.